GRAFIX 2.70

Tandy 1000/PCjr Graphics And Sound Extensions

160x200x16 Color Graphics Mode

320x200x16 Color Graphics Mode

640x200x16 Color Graphics Mode

Texas Instruments SN76496 Sound Chip

Written By:

Joseph A. Albrecht

Table of contents:

1 - Introduction To The GRAFIX Package

2 - BASIC And PASCAL Programming Interface

3 - ASSEMBLER Programming Interface

4 - GRAFIX Utility Programs

5 - Miscellaneous Details About The GRAFIX Package

CHAPTER 1 - Introduction

GRAFIX is a memory resident graphics and sound program for your Tandy 1000/PCjr

series personal  computer. It provides you  with a complete set  of graphics and

sound  functions that  can be  executed from  your favorite programming language

such as compiled BASIC and PASCAL. To  use GRAFIX just install it when you start

up your computer by including the following lines in your AUTOEXEC.BAT file:

TANDY11

GRAFIX

That's all there is to it. Since the program is memory resident you can use any

of the program's sound and graphic functions anytime you want. The TANDY11

program is necessary if own a Tandy 1000 SL/TL/RL computer. This enables those

computers to do 640x200x16 color graphics. (See chapter 4 for more

information on the TANDY11 program)

The following list are some of the functions GRAFIX provides you with:

Points

Lines

Circles and ellipses

Line and solid boxes

Filling irregular areas with color

Storing and transferring areas of the graphic screen

Variable screen viewport

Multiple video pages

Printing text strings on the graphics screen

Graphics text cursor

Using the Tandy 1000/PCjr's 3 voice sound and noise capabilities

Making sounds and noises in background while your program executes

Changing the speed of the system timer for precise sound and noise duration

Three different graphic modes supported:

160x200x16 color graphics

320x200x16 color graphics

640x200x16 color graphics (Tandy 1000 SL/TL/RL only)

These functions are accessed by passing  the GRAFIX program a set values through

your computer's  CPU registers and  executing an INT  instruction. Sounds pretty

easy, huh! Well, you  never have to be that concerned with  the mechanics of the

GRAFIX  program. I  have provided  you with  a set  of interfacing  programs for

compiled BASIC  and PASCAL. These  interfaces act as  a translator between  your

program and the GRAFIX program. For example, if you wanted to draw a blue box in

Turbo PASCAL all you have to do is include a line like this in your program:

DrawBox(0, 0, 319, 199, 1);

The interface  takes the values  passed to  it,  places the values  into the CPU

registers, and  executes the INT  instruction without you  ever having to  worry

about  those  intricate  details.  The  following  is  list  of  interfaces  for

programming languages currently provided:

Microsoft QuickBASIC 4.0 and greater

Borland Turbo BASIC 1.0 - 1.1

SPECTRA Publishing PowerBASIC 2.0 - 2.1

Microsoft QuickPASCAL 1.0 and greater

Borland Turbo PASCAL 4.0 and greater

Chapter 2 describes the BASIC and PASCAL programming interfaces telling you what

each function does and how to use it  in your program. For those of you who have

a different programming  language and would  like to design  your own interface,

read chapter  3. This describes the  ASSEMBLY language interface for  GRAFIX. It

gives all  the technical information necessary  for setting up an  interface for

any other  programming language such as  C. Chapter 4 discusses  the two utility

programs, ADJMEM and  TANDY11 provided with the GRAFIX  package. Chapter 5 gives

all the miscellaneous details of GRAFIX.

CHAPTER 2 - BASIC And PASCAL Programming Interfaces

This chapter describes  the BASIC and PASCAL programming  interfaces for GRAFIX.

All the  functions are listed in  alphabetical order with a  description of what

the function  does, an explanation of  each parameter, and an  example of how to

use  it in  your program  for  both  BASIC and  PASCAL. Following  the functions

listing there is a section on the necessary steps to integrate the interface for

each language product into your program.

Functions summary for BASIC and PASCAL:

********************************************************************************

Animate(X, Y, Image[Index], Duration)

Animates an image as described in the 'ExtPut' function.

It  automatically  transfers  and  erases  the  image  all  in one step. The

duration  is the  amount of  time to  pause between  the transfer  and erase

stages of the function. This can help  adjust the amount of flicker. (Do not

confuse the duration for this function with the duration for the 'ExtSound',

'Noise', and 'Delay' functions. The duration here is in no way tied to clock

ticks or the 8253 timer chip)

Parameters:

X - Horizontal position of image

Y - Vertical position of image

Image[Index] - Integer array  holding saved image starting  at the specified

index

Duration is a number 1 - 65535:

This is the amount of delay between transfer and erase of image

Examples:

BASIC

CALL Animate(X, Y, Image%(0), Duration%)

PASCAL

Animate(X, Y, Image[0], Duration);

********************************************************************************

AudioOff

Turns all current sounds and noises off and disables background processing.

See the 'ExtSound' and 'Noise'  functions for more information on background

processing.

Parameters:

None

Examples:

BASIC

CALL AudioOff

PASCAL

AudioOff;

********************************************************************************

BLoad(FileName, BLoadArea)

Stores a file  saved with BASIC's BSAVE function into  the specified area of

memory.

This is available  for PASCAL programs only. This  is primarily intended for

use with the demo programs provided with the GRAFIX package in which BSAVE'd

images  are  stored  into  arrays.  Since  PASCAL  lacks  a BLOAD function I

provided one to fill in the gap.

Parameters:

FileName - Name of BSAVE'd file

BLoadArea  - Area  of memory  to  store  BSAVE'd image.  This is  usually an

integer array.

Examples:

BASIC

None

PASCAL

BLoad('IMAGE.BIN', Image[0]);

********************************************************************************

ClearKey

Clears the keyboard buffer.

Parameters:

None

Examples:

BASIC

CALL ClearKey

PASCAL

ClearKey;

********************************************************************************

ClearScreen

Clears the screen viewport using the current background color.

See the 'SetView' function for a description of the screen viewport.

Parameters:

None

Examples:

BASIC

CALL ClearScreen

PASCAL

ClearScreen;

********************************************************************************

ColorOff

Turns the color signal off for use with a monochrome composite monitor.

Parameters:

None

Examples:

BASIC

CALL ColorOff

PASCAL

ColorOff;

********************************************************************************

ColorOn

Turns  the  color  signal  back  on  again  if  you  had  turned it off with

'ColorOff'.

Parameters:

None

Examples:

BASIC

CALL ColorOn

PASCAL

ColorOn;

********************************************************************************

CurrentPoint(X, Y)

Returns the current X, Y coordinates.

Parameters:

X - Variable to hold current horizontal position

Y - Variable to hold current vertical position

Examples:

BASIC

CALL CurrentPoint(X%, Y%)

PASCAL

CurrentPoint(X, Y);

********************************************************************************

CursorOff

Sets the graphics cursor off making it invisible.

Parameters:

None

Examples:

BASIC

CALL CursorOff

PASCAL

CursorOff;

********************************************************************************

CursorOn

Sets the graphics cursor on making it visible.

Parameters:

None

Examples:

BASIC

CALL CursorOn

PASCAL

CursorOn;

********************************************************************************

CursorStatus(Status, Size)

Returns the current graphics cursor status and size.

Parameters:

Status - Variable to hold graphics cursor status

0 = Off

1 =  On

SizeStatus - Variable to hold graphics cursor size

0 = Normal Size

1 = Half Size

Examples:

BASIC

CALL CursorStatus(Status%, SizeStatus%)

PASCAL

CursorStatus(Status, SizeStatus);

********************************************************************************

DrawBox(X1, Y1, X2, Y2, Color)

Draws a line box from X1, Y1 to X2, Y2 using the specified color.

Parameters:

X1 - Upper left horizontal position of box

Y1 - Upper left vertical position of box

X2 - Lower right horizontal position of box

Y2 - Lower right vertical position of box

Color - Color of box

Examples:

BASIC

CALL DrawBox(0, 0, 319, 199, 1)

PASCAL

DrawBox(0, 0, 319, 199, 1);

********************************************************************************

DrawBoxC(X1, Y1, X2, Y2)

Draws a line box from X1, Y1 to X2, Y2 using the current drawing color.

Parameters:

X1 - Upper left horizontal position of box

Y1 - Upper left vertical position of box

X2 - Lower right horizontal position of box

Y2 - Lower right vertical position of box

Examples:

BASIC

CALL DrawBoxC(10, 10, 100, 100)

PASCAL

DrawBoxC(10, 10, 100, 100);

********************************************************************************

ExitGraphics

Leaves the  graphics screen mode, enters  the 80x25 color text  screen mode,

and disables all graphics functions.

Parameters:

None

Examples:

BASIC

CALL ExitGraphics

PASCAL

ExitGraphics;

********************************************************************************

Ellipse(X, Y, MajorAxis, MinorAxis)

Draws an ellipse centered at the specified X, Y position using the specified

major and minor axes and current drawing color.

When you draw a  circle you specify an X, Y center  along with a radius. The

radius  is a  single number  that indicates  how many  points away  from the

center the circle should expand out. The  radius is in fact broken down into

two  separate numbers;  a number  of horizontal  and vertical  points. These

numbers  are derived  from the  current  setting  of the  aspect ratio.  The

horizontal and vertical number of points are called the major and minor axes

of the ellipse.  Using the aspect ratio to break  down the radius into these

two  parts has  it's limitations.   Calling the  ellipse procedure  with the

specified major  and minor axes  gives you the  ultimate control in  drawing

circular shapes.

Parameters:

X - Horizontal center of ellipse

Y - Vertical center of ellipse

MajorAxis - Size of major axis

MinorAxis - Size of minor axis

Examples:

BASIC

CALL Ellipse(160, 100, 50, 40)

PASCAL

Ellipse(160, 100, 50, 40);

********************************************************************************

EllipseC(X, Y, MajorAxis, MinorAxis, Color)

Draws an ellipse centered at the specified X, Y position using the specified

major and minor axes and specified color.

See the 'Ellipse' function for a description of an ellipse.

Parameters:

X - Horizontal center of ellipse

Y - Vertical center of ellipse

MajorAxis - Size of major axis

MinorAxis - Size of minor axis

Color - Color of ellipse

Examples:

BASIC

CALL EllipseC(160, 100, 50, 40, 1)

PASCAL

EllipseC(160, 100, 50, 40, 1);

********************************************************************************

ExtCircle(X, Y, Radius)

Draws a circle  centered at X, Y using the  specified radius, current values

of aspect ratio and current drawing color.

See the 'SetAspect' function for a description of the aspect ratio.

Parameters:

X - Horizontal center of circle

Y - Vertical center of circle

Radius - Radius of circle

Examples:

BASIC

CALL ExtCircle(160, 100, 75)

PASCAL

ExtCircle(160, 100, 75)

********************************************************************************

ExtCircleC(X, Y, Radius, Color)

Draws a circle  centered at X, Y using the  specified radius, current values

of aspect ratio and the specified color.

See the 'SetAspect' function for a complete description of the aspect ratio.

Parameters:

X - Horizontal center of circle

Y - Vertical center of circle

Radius - Radius of circle

Color - Color of circle

Examples:

BASIC

CALL ExtCircleC(160, 100, 75, 1)

PASCAL

ExtCircle(160, 100, 75, 1)

********************************************************************************

ExtGet(X1, Y1, X2, Y2, Image[Index])

Stores a graphics image from X1, Y1 to X2, Y2 into the an integer array.

This is used  with the 'ExtPut' function to store  and transfer areas of the

graphics  screen. Be  EXTREMELY careful  when using  this function in making

sure the array is large enough to  hold the entire image you are storing. If

not,  this  function  will  cheerfully  overwrite  anything after the ending

position of  the array in  your program. This  includes other variables,  or

even  program instructions.  The result   would be  less than  pleasurable I

assure you. Fortunately, there is a simple solution to this problem that can

be set up in your program to  see that this terrible event never occurs. Use

the following procedure to determine if  the array is large enough to safely

store the whole image:

SafeSize = ((W / 4) + 6) * H

H - Image's number of points in height

W - Image's number of points in width

ArraySize = L - S + 1

L - Integer array's largest possible index number

S - Integer array's smallest possible index number

IF ArraySize < SafeSize THEN DON'T execute this function

Most programming  languages allow you  to determine an  array's smallest and

largest index number.  If the language you are working  in doesn't have this

capability then you'll have to supply those figures yourself.

Parameters:

X1 - Upper left horizontal position to store

Y1 - Upper left vertical position to store

X2 - Lower right horizontal position to store

Y2 - Lower right vertical position to store

Image[Index] - Integer array to hold graphic image starting at the specified

index

Examples:

BASIC

CALL ExtGet(0, 0, 50, 75, Image%(0))

PASCAL

ExtGet(0, 0, 50, 75, Image[0]);

********************************************************************************

ExtLine(X1, Y1, X2, Y2)

Draws a line from X1, Y1 to X2, Y2 using the current drawing color.

Parameters:

X1 - Upper left horizontal position of line

Y1 - Upper left vertical position of line

X2 - Lower right horizontal position of line

Y2 - Lower right vertical position of line

Examples:

BASIC

CALL ExtLine(10, 15, 100, 150)

PASCAL

ExtLine(10, 15, 100, 150);

********************************************************************************

ExtLineC(X1, Y1, X2, Y2, Color)

Draws a line from X1, Y1 to X2, Y2 using the specified color.

Parameters:

X1 - Upper left horizontal position of line

Y1 - Upper left vertical position of line

X2 - Lower right horizontal position of line

Y2 - Lower right vertical position of line

Color - Color of line

Examples:

BASIC

CALL ExtLineC(10, 15, 100, 150, 1)

PASCAL

ExtLineC(10, 15, 100, 150, 1);

********************************************************************************

ExtPaint(X, Y, FillColor, BoundaryColor)

Fills  an enclosed  shape starting  at X,  Y with  the specified  fill color

stopping at the specified boundary color.

An example of using  the ExtPaint function would be if you  drew a circle in

red and wanted  to fill the interior with  a blue. If you gave  the starting

position  as the  center of  the circle,  the fill  color as  blue, and  the

boundary color as red; the interior  would be filled with blue while leaving

the red border of the circle alone.

Parameters:

X - Starting horizontal position to paint

Y - Starting vertical position to paint

FillColor - Color used to fill the paint area

BoundaryColor - Color at which to stop painting

Examples:

BASIC

CALL ExtPaint(100, 10, 5, 14)

PASCAL

ExtPaint(100, 10, 5, 14);

********************************************************************************

ExtPut(X, Y, Image[Index], PutAction)

Transfers a  graphics image stored with  'ExtGet' starting at X,  Y using the

specified action.

Action is 1 of 5 options described as follows:

1 = PRESET - Produces a numeric opposite of the color stored

in the array:

Array Color   Screen Color

-----------   ------------

00             15

01             14

02             13

03             12

04             11

05             10

06             09

07             08

08             07

09             06

10             05

11             04

12             03

13             02

14             01

15             00

2 = PSET - Transfers the image exactly as it was stored.

3 = AND - Does a logical AND  between the point on the screen and the color

in the array.

4 = OR - Does a logical OR between the point on the screen and the color in

the array.

5 = XOR - Does a logical XOR between the  point on the screen and the color

in the array. When PUT against a background twice, the background

is  restored. This  allows animation  to take  place. To  perform

animation do the following:

1) PUT the image on the screen

2) Calculate the next X, Y position

3) PUT the image at the previous X, Y position

4) Repeat Step 1

Parameters:

X - Horizontal position to transfer image

Y - Vertical position to transfer image

Image[Index] - Integer array holding graphic image starting at the specified

index

PutAction - Action to use when transferring image

Examples:

BASIC

CALL ExtPut(10, 20, Image%(0), 5)

PASCAL

ExtPut(10, 20, Image[0], 5);

********************************************************************************

ExtSound(Frequency, Duration, Volume, Channel)

Generates a sound with the  specified frequency, duration, volume, and sound

channel.

This function works exactly like the SOUND function in the special version

of GW-BASIC that came with you Tandy 1000/PCjr. Sounds can be made in

background if you enable sound buffering. This means that your program will

continue to execute while sounds are being made. Each sound channel has

it's own sound buffer. Each buffer can hold up to 64 entries.

Parameters:

Frequency is a number 110 - 32767:

This is  the tone to produce  in Hertz. Certain frequencies  correspond to

musical  notes.   The  following  chart  shows   the  frequency  for  each

corresponding note:

Note  Freq     Note  Freq     Note  Freq     Note  Freq

----  ----     ----  ----     ----  ----     ----  ----

C2      65     A3     220     F#5    740     C7    2489

C#2     69     A#3    233     G5     784     C#7   2637

D2      73     B3     247     G#5    831     D7    2794

D#2     78     C4     262     A5     880     D#7   2960

E2      82     C#4    277     A#5    932     E7    3136

F2      87     D4     294     B5     988     F7    3322

F#2     93     D#4    311     C6    1047     F#7   3520

G2      98     E4     330     C#6   1109     G7    3729

G#2    104     F4     349     D6    1175     G#7   3951

A2     110     F#4    370     D#6   1245     A7    4186

A#2    116     G4     392     E6    1319     A#7   4435

B2     123     G#4    415     F6    1397     B7    4699

C3     131     A4     440     F#6   1480     C8    4978

C#3    139     A#4    466     G6    1568     C#8   5274

D3     147     B4     494     G#6   1661     D8    5587

D#3    156     C5     523     A6    1760     F#8   5919

E3     165     C#5    554     A#6   1865     G8    6271

F3     175     D5     587     B6    1976     G#8   6645

F#3    185     D#5    622     D#8   2093     A8    7040

G3     196     E5     659     E8    2217     A#8   7459

G#3    208     F5     698     F8    2349     B8    7902

The Texas Instruments SN76496 sound chip  in the Tandy 1000 cannot produce

frequencies below  110 Hertz. If you  use the above chart  please remember

you cannot make notes below A2.

Duration is a number 0 - 65535:

This is the number of clock ticks the sound will last. See the 'FastTimer'

function for more information on clock ticks.

Volume is a number 0 - 15:

0 is the quietest.

15 is the loudest.

Sound channel is a number 0 - 2:

This is  the sound channel  to use when  producing the sound.  You can set

each sound channel to a different tone to make a 3-note chord.

Examples:

BASIC

CALL ExtSound(880, 8, 7, 0)

PASCAL

ExtSound(880, 8, 7, 0);

********************************************************************************

FastTimer(Speed)

Sets the speed of the 8253 timer chip.

The 8253 timer chip is the driving force behind the 'ExtSound', 'Noise', and

'Pause' functions. These functions rely on a certain period of time to last.

This period of time  is known as a clock tick. When  you first start-up your

computer  the  timer  chip  is  set  at  18.2  clock  ticks  per second. The

'FastTimer' function allows  you to change the speed to  a rate greater than

that. The  reason for this is  to increase the performance  of the functions

previously mentioned. The 'ExtSound' function is a good example. You can use

the 'ExtSound' function to produce notes  to make music. There is a limiting

factor in  that at a  rate of 18.2  clock ticks per  second notes that  last

1/19th or less their normal time cannot be produced. GW-BASIC sets the timer

chip to  run at 4  times it's normal  rate when it  uses the PLAY statement.

This allows it to make notes up to 1/64th their normal time. The 'FastTimer'

function allows you to run the timer  chip up to 256 times it's normal rate.

This gives  you almost unlimited  flexibility in writing  programs that make

sounds or  noises to last  exactly as long  as you want.  Another area where

this function can  be useful is in the  use of the 'Pause' function.  A more

precise time than 1/18th of a second can be used if you need it. I recommend

that you use values of 8 or less  if you are going to use this function only

for sounds or  noises and set it to  rates greater than that for  only short

periods of time in the use of  the 'Pause' function. This is because running

the 8253 timer  faster than it's normal setting will  cause your computer to

run slower. The higher the rate setting the slower your computer will run.

An extremely important  thing to remember about using  this function is that

you MUST use the 'ResetTimer' function  before you end your program. Failure

to do so can result in anything from your current time of day being wrong to

a locked up computer system. Avoid using  this function if you are using the

debugging facilities  of one of the  compilers. As soon as  a break point is

reached the program  is ended for all practical purposes.  The main thing to

remember is the 'ResetTimer' timer  function must be executed before control

is passed back to the operating system or the compiler you are working with.

Parameters:

Speed is a number 2 - 256:

This is the number  of times greater you want the 8253  timer chip to run.

A setting of 4 will make it run 4 times faster than before. This will make

clock ticks occur 72.8 times a second.

Examples:

BASIC

CALL FastTimer(8)

PASCAL

FastTimer(4);

********************************************************************************

FillBox(X1, Y1, X2, Y2, Color)

Draws a solid box from X1, Y1 to X2, Y2 using the specified color.

Parameters:

X1 - Upper left horizontal position of box

Y1 - Upper left vertical position of box

X2 - Lower right horizontal position of box

Y2 - Lower right vertical position of box

Color - Color of box

Examples:

BASIC

CALL FillBox(0, 0, 100, 75, 1)

PASCAL

FillBox(0, 0, 100, 75, 1);

********************************************************************************

FillBoxC(X1, Y1, X2, Y2)

Draws a solid box from X1, Y1 to X2, Y2 using the current drawing color.

Parameters:

X1 - Upper left horizontal position of box

Y1 - Upper left vertical position of box

X2 - Lower right horizontal position of box

Y2 - Lower right vertical position of box

Examples:

BASIC

CALL FillBoxC(0, 0, 100, 75)

PASCAL

FillBoxC(0, 0, 100, 75);

********************************************************************************

GetActivePage(Page)

Returns  the  current  active  page.  See  the  'PageCopy' function for more

information on video paging.

Parameters:

Page - Variable to store the value of the current active page.

Examples:

BASIC

CALL GetActivePage(Page%)

PASCAL

GetActivePage(Page);

********************************************************************************

GetAspect(XAspect, YAspect)

Returns  the current  horizontal and  vertical portions  of the aspect ratio

used in circle drawing.

Parameters:

XAspect - Variable to store horizontal portion of aspect ratio

YAspect - Variable to store vertical portion of aspect ratio

Examples:

BASIC

CALL GetAspect(XAspect%, YAspect%)

PASCAL

GetAspect(XAspect, YAspect);

********************************************************************************

GetBackColor(Color)

Returns the current background color.

Parameters:

Color - Variable to store current background color

Examples:

BASIC

CALL GetBackColor(BackColor%)

PASCAL

GetBackColor(BackColor);

********************************************************************************

GetCursor(Row, Column)

Returns the current text row and column.

Parameters:

Row - Variable to store current text row

Column - Variable to store current text column

Examples:

BASIC

CALL GetCursor(Row%, Column%)

PASCAL

GetCursor(Row, Column);

********************************************************************************

GetDisplayPage(Page)

Returns  the current  display page.   See the  'PageCopy' function  for more

information on video paging.

Parameters:

Page - Variable to store the value of the current display page.

Examples:

BASIC

CALL GetDisplayPage(Page%)

PASCAL

GetDisplayPage(Page);

********************************************************************************

GetInput(Prompt, String)

Allows input of string variables while in the color graphics mode.

It provides many of the editing  capabilities of BASIC's INPUT function. The

keys that are listed show what editing functions are currently handled:

BACKSPACE - Backspaces  the cursor,  erasing  the  first character  to the

left. All characters to right move left one position.

ENTER - Exits the  'GetInput' function and  transfers the entered  data to

string variable.

ESC - Erases  the  entire  line  and  moves  the  cursor  to  first keying

position.

HOME - Moves the cursor to the first keying position.

LEFT ARROW - Moves the cursor left one space.

RIGHT ARROW - Moves the cursor right one space

END - Moves the end of the line.

INSERT - Toggles insert mode on/off. If  insert mode is on then characters

are added at current cursor position.  If insert mode is off then

characters are overwritten at the current cursor position.

DELETE - Deletes the  character at the  current cursor position  and moves

all remaining characters to the left one position.

The graphics cursor is turned on  for this function. It's previous status is

restored when the function is done.

Parameters:

Prompt - Prompt to display for input

String - String variable to hold input entered

Examples:

BASIC

CALL GetInput("Enter Your Name ", A$)

PASCAL

GetInput('Enter Your Name ', A);

********************************************************************************

GetPlotColor(Color)

Returns the current drawing color.

Parameters:

Color - Variable to store current drawing color

Examples:

BASIC

CALL GetPlotColor(PlotColor%)

PASCAL

GetPlotColor(PlotColor);

********************************************************************************

GetScreen(Image[Index])

Copies the entire screen buffer into an integer array.

This method is at least 5 times faster than using the 'ExtGet' function.

Parameters:

Image[Index] - The  integer array to hold the screen  buffer at the starting

index.

To make  sure the integer  array is large  enough to hold  the entire screen

buffer use  the following chart  for the proper  array size for  each screen

mode:

160x200x16 color graphics mode:

BASIC

DIM Image%(8005)

PASCAL

Image: ARRAY[0..8005] OF WORD;

320x200x16 color graphics mode:

BASIC

DIM Image%(16005)

PASCAL

Image: ARRAY[0..16005] OF WORD;

640x200x16 color graphics mode:

BASIC

DIM Image%(32005)

PASCAL

Image: ARRAY[0..32005] OF WORD;

Examples:

BASIC

CALL GetScreen(Image%(0))

PASCAL

GetScreen(Image%[0]);

********************************************************************************

GetTandy11(Status)

Returns the the  status of the TANDY11 driver for  using the 640x200x16 color

graphics mode on the Tandy 1000 SL/TL computers.

Parameters:

Status - Variable to hold status of Tandy11 driver

0 = TANDY11 driver is not installed

1 = TANDY11 driver is installed

Examples:

BASIC

CALL GetTandy11(Tandy11%)

PASCAL

GetTandy11(Tandy11);

********************************************************************************

GetTextColor(Color)

Returns the current text color used in the 'PrintString' function.

Parameters:

Color - Variable to store current text color.

Examples:

BASIC

CALL GetTextColor(TextColor%)

PASCAL

GetTextColor(TextColor);

********************************************************************************

GetTimer(TimerValue)

Returns  the current  speed of  the  8253  timer chip.  See the  'FastTimer'

function for more information on the 8253 timer chip.

Parameters:

TimerValue - Variable to store the speed of the 8253 timer chip.

Examples:

BASIC

CALL GetTimer(TimerValue%)

PASCAL

GetTimer(TimerValue);

********************************************************************************

GetView(MinX, MinY, MaxX, MaxY)

Returns the current values of the screen viewport.

Parameters:

MinX - Variable to hold minimum X value

MinY - Variable to hold minimum Y value

MaxX - Variable to hold maximum X value

MaxY - Variable to hold maximum Y value

Examples:

BASIC

CALL GetView(MinX%, MinY%, MaxX%, MaxY%)

PASCAL

GetView(MinX, MinY, MaxX, MaxY);

********************************************************************************

GraphStatus(Status)

Returns whether the graphics functions are enabled or disabled.

Parameters:

Status - Variable to hold graphic functions status

0 = Graphic functions are disabled

1 = Graphic functions are enabled

Examples:

BASIC

CALL GraphStatus(Status%)

PASCAL

GraphStatus(Status);

********************************************************************************

HalfCursor

Sets the text cursor to half size.

Parameters:

None

Examples:

BASIC

CALL HalfCursor

PASCAL

HalfCursor;

********************************************************************************

HighGraphics

Enters the  640x200x16 color graphics  screen mode and  enables all graphics

functions on the Tandy 1000 SL/TL computers only.

This function verifies both the GRAFIX and TANDY11 drivers are loaded before

attempting to enter the graphics mode.

Parameters:

None

Examples:

BASIC

CALL HighGraphics

PASCAL

HighGraphics;

********************************************************************************

LineRel(XRel, YRel)

Draws a  line a relative distance  from the current X,  Y position using the

current drawing color.

You can give this function both positive and negative values.

Parameters:

XRel - Number points to draw line in a horizontal direction

YRel - Number points to draw line in a vertical direction

Examples:

BASIC

CALL LineRel(10, 10)

PASCAL

LineRel(10, 10);

********************************************************************************

LineRelC(XRel, YRel, Color)

Draws a  line a relative distance  from the current X,  Y position using the

specified color.

You can give this function both positive and negative values.

Parameters:

XRel - Number points to draw line in a horizontal direction

YRel - Number points to draw line in a vertical direction

Color - Color of line

Examples:

BASIC

CALL LineRelC(-5, -10, 15)

PASCAL

LineRelC(-5, -10, 15);

********************************************************************************

LineTo(X, Y)

Draws a line to  the specified X, Y position from the  current X, Y position

using the current drawing color.

Parameters:

X - Ending horizontal position of line

Y - Ending vertical position of line

Examples:

BASIC

CALL LineTo(50, 75)

PASCAL

LineTo(50, 75);

********************************************************************************

LineToC(X, Y, Color)

Draws a line to  the specified X, Y position from the  current X, Y position

using the specified drawing color.

Parameters:

X - Ending horizontal position of line

Y - Ending vertical position of line

Color - Color of line

Examples:

BASIC

CALL LineToC(50, 75, 6)

PASCAL

LineToC(50, 75, 6);

********************************************************************************

LowGraphics

Enters the  160x200x16 color graphics  screen mode and  enables all graphics

functions.

This  function verifies  the GRAFIX  driver is  loaded before  attempting to

enter the graphics mode.

Parameters:

None

Examples:

BASIC

CALL LowGraphics

PASCAL

LowGraphics;

********************************************************************************

MediumGraphics

Enters the  320x200x16 color graphics  screen mode and  enables all graphics

functions.

This  function verifies  the GRAFIX  driver is  loaded before  attempting to

enter the graphics mode.

Parameters:

None

Examples:

BASIC

CALL MediumGraphics

PASCAL

MediumGraphics;

********************************************************************************

MoveRel(X, Y)

Moves a  relative distance from the  current X, Y position  without plotting

any points.

You can give this function both positive and negative values.

Parameters:

X - Number of points to move in a horizontal direction

Y - Number of points to move in a vertical direction

Examples:

BASIC

CALL MoveRel(15, 20)

PASCAL

MoveRel(15, 20);

********************************************************************************

MoveTo(X, Y)

Moves to the specified X, Y position  from the current X, Y position without

plotting any points.

Parameters:

X - Horizontal position to move to

Y - Vertical position to move to

Examples:

BASIC

CALL MoveTo(0, 0)

PASCAL

MoveTo(0, 0);

********************************************************************************

Noise(Source, Volume, Duration)

Generates a periodic or white noise  sound with the specified volume for the

specified length of time.

This function works  exactly like the NOISE function  in the special version

of GW-BASIC  that came with  your Tandy 1000/PCjr.  Test it out  to hear the

difference between the periodic and white noise sound effects. Noises can be

made  in background  if you  enable noise  buffering. This  means that  your

program will continue to execute while noises are being made. The noise

buffer can hold up to 64 entries.

Parameters:

Source is a number 0-7:

0-3 are periodic noises

4-7 are white noises

Volume is a number 0-15:

0 is the quietest

15 is the loudest

Duration is a number 0 to 65535:

This is the number of clock ticks the noise will last. See the 'FastTimer'

function for more information on clock ticks.

Examples:

BASIC

CALL Noise(4, 8, 12)

PASCAL

Noise(4, 8, 12);

********************************************************************************

NoiseBufOff

Disables background processing of noises.

See the 'Noise' function for a description of background noise processing.

Parameters:

None

Examples:

BASIC

CALL NoiseBufOff

PASCAL

NoiseBuffOff

********************************************************************************

NoiseBufOn

Enables the processing of noises in background.

See the 'Noise' function for a description of background noise processing.

Parameters:

None

Examples:

BASIC

CALL NoiseBufOn

PASCAL

NoiseBufOn;

********************************************************************************

NoiseOff

Turns  off all  current noises   off and  disables background  processing of

noises.

See the 'Noise' function for a description of background noise processing.

Parameters:

None

Examples:

BASIC

CALL NoiseOff

PASCAL

NoiseOff

********************************************************************************

NormalCursor

Sets the text cursor to normal size.

Parameters:

None

Examples:

BASIC

CALL NormalCursor

PASCAL

NormalCursor;

********************************************************************************

PageCopy(Source, Destination)

Copies the source video page to the destination video page.

The following is a table showing the video modes that GRAFIX supports on the

Tandy 1000:

Memory    Video   Page #

Screen Size   Colors   Required   Pages   Range

-----------   ------   --------   -----   ------

160x200       16       16K        8      0-7

320x200       16       32K        4      0-3

640x200       16       64K        2      0-2 (Tandy 1000 SL/TL only)

The memory required  is the amount of memory needed  for one graphic screen.

The Tandy 1000/PCjr has the ability to access up to 128K of video memory. As

you can see  each graphic screen is less  than 128K in size. The  Tandy 1000

allows you to select  which part of that 128K you want  to work with as well

as which part you want to display on your monitor. These graphic screens are

known  as video  pages. You  can work  with one  video page while displaying

another. There  are two types of  video pages, a Display  page and an Active

page. The  Display page is the  one that shows up  on your computer monitor.

The Active  page is the one  that all the graphic  functions will work with.

The two  do not have to  be the same. You  can for example work  with page 1

while displaying page 0. When you initially enter any of the video modes the

Active  and Display  pages are  both set  to 0.  You can  change these  with

'SetActivePage' and  'SetDisplayPage' functions. The  number of video  pages

you have to work with depends on  the video mode you choose. The above table

shows  the maximum  number of  video  pages  for each  video mode.  One very

important  thing to  remember before  using video  pages is  that the  Tandy

1000's video memory MUST first be protected. See the 'ADJMEM' section in

chapter 4 for an explanation of why this must be done.

Parameters:

Source - The video page to copy from.

Destination - The video page to copy to.

Examples:

BASIC

CALL PageCopy(1, 0)

PASCAL

PageCopy(0, 1);

********************************************************************************

PaletteUsing(PaletteArray[Index])

Changes the  color palette using  16 elements of  an integer array  starting

with the specified index value.

Each array element  holds the new color value  for the corresponding palette

entry. See the 'SetPalette' function for a description of palette registers.

Parameters:

PaletteArray[Index] - Integer holding the  palette entries starting with the

specified index.

Examples:

BASIC

CALL PaletteUsing(PalArray%(Index%))

PASCAL

PaletteUsing(PalArray[Index]);

********************************************************************************

Pause(Delay)

Delays a program for the specified number of clock ticks.

Clock  ticks occur  at a  default rate  of 18.2  times a  second unless  the

'FastTimer'  function  is  used.  A  number  of  36  would  delay  a program

approximately 2  seconds at the default  clock tick rate. This  function has

the  advantage being  more precise  than a  high level  language delay  loop

because  it uses  an assembly  language interface.  Use it  when you need to

delay for a precise number of clock ticks.

Parameters:

Delay - Number of clock ticks to delay program. See the 'FastTimer' function

for more information on clock ticks.

Examples:

BASIC

CALL Pause(20)

PASCAL

Pause(20);

********************************************************************************

PlotPoint(X, Y)

Plots  a point  at the  specified X,  Y position  using the  current drawing

color.

Parameters:

X - Horizontal position to plot point

Y - Vertical position to plot point

Examples:

BASIC

CALL PlotPoint(160, 100)

PASCAL

PlotPoint(160, 100);

********************************************************************************

PlotPointC(X, Y, Color)

Plots a point at the specified X, Y position using the specified color.

Parameters:

X - Horizontal position to plot point

Y - Vertical position to plot point

Color - Color of point

Examples:

BASIC

CALL PlotPoint(160, 100, 5)

PASCAL

PlotPoint(160, 100, 5);

********************************************************************************

PointColor(X, Y, Color)

Returns the color of the point at the specified X, Y position.

Parameters:

X - Horizontal position of point

Y - Vertical position of point

Color - Variable to hold color of point

Examples:

BASIC

CALL PointColor(100, 100, PixelColor%)

PASCAL

PointColor(100, 100, PixelColor);

********************************************************************************

PrintString(String)

Prints a text  string using the current text color  and advances to the next

line.

Parameters:

String - Text string to print

Examples:

BASIC

CALL PrintString("Hi there everybody")

PASCAL

PrintString('Hi there everybody');

********************************************************************************

PrintStringX(String)

Prints a  text string using the  current text color but  does not advance to

the next line.

Parameters:

String - Text string to print

Examples:

BASIC

CALL PrintStringX("Hi there everybody")

PASCAL

PrintStringX('Hi there everybody');

********************************************************************************

PutScreen(Image[Index])

Transfers the saved image from an  integer array stored with the 'GetScreen'

function to the screen buffer.

This method is 5 times faster than using the 'ExtPut' function.

Parameters:

Image[Index] - The  integer array holding the screen  buffer at the starting

index.

Examples:

BASIC

CALL PutScreen(Image%(0))

PASCAL

PutScreen(Image%[0]);

********************************************************************************

ResetPalette

Resets the color palette to the default state.

See the 'SetPalette' function for a description of palette registers.

Parameters:

None

Examples:

BASIC

CALL ResetPalette

PASCAL

ResetPalette;

********************************************************************************

ResetTimer

Resets the 8253  timer chip to it's default speed  of 1. See the 'FastTimer'

function for a description of the 8253 timer chip.

Parameters:

None

Examples:

BASIC

CALL ResetTimer

PASCAL

ResetTimer;

********************************************************************************

ResetView

Resets the screen viewport to it's default size.

See the SetView function for a description of the screen viewport.

Parameters:

None

Examples:

BASIC

CALL ResetView

PASCAL

ResetView;

********************************************************************************

SetActivePage(ActivePage)

Sets the  current active page which  all graphics functions will  work with.

See the 'PageCopy' function for a description of video pages.

Parameters:

ActivePage - Active page to set.

Examples:

BASIC

CALL SetActivePage(ActivePage%)

PASCAL

SetActivePage(ActivePage);

********************************************************************************

SetAspect(XAspect, YAspect)

Sets the horizontal and vertical portions of the aspect ratio used in circle

drawing.

The three different video modes that the GRAFIX program supports each have a

different number  of points horizontally  than vertically. For  example, the

640x200x16  color graphics  mode has  640 points  horizontally by 200 points

vertically. Another factor  is the computer monitor you  are using. Computer

monitors  are just  like televisions  sets in  that ratio  to the  number of

points horizontally  to the number  of points vertically  is 4:3. These  two

factors mean that when you draw a  shape on the graphics screen it will look

out  of proportion  to it's  size. For  example a  100x100 box  drawn on the

640x200 graphics screen would like approximately like this:

********

*      *

*      *

*      *

*      *

*      *

*      *

*      *

*      *

*      *

*      *

********

The box  is 100 points high  by 100 points wide,  yet it is not  square. Any

shape  drawn in  any one  of the  three graphics  mode will  suffer the same

result unless  the points are scaled  in proportion to the  4:3 ratio of the

computer monitor and to the number of horizontal and vertical points for the

graphics  mode being  used. The  scaling factor  used in  drawing circles is

called the aspect ratio. This is calculated by dividing the number of points

horizontally by the number of points vertically. Divide this number by 4 and

then multiply  by 3. The aspect  ratio for the 640x200  graphics mode is 2.4

((640 / 200) / 4) * 3). There is a problem in that the GRAFIX program cannot

accept fractional numbers as a parameter.  The solution here is to break the

aspect ratio into two parts: a horizontal portion and vertical portion. This

can be done for any graphics mode  by dividing down the number of horizontal

and vertical  points until they  both cannot be  divided evenly anymore  and

scaling the horizontal portion. The 640x200 mode would work like this:

640   64   16   16 / 4   4   4 * 3   12

--- : -- : -- : --     : - : -     : --

200   20    5    5       5   5        5

The  final  answer  is  12:5.  First  we  divided  640:200 down to 16:5. The

horizontal portion  was then scaled by  dividing by 4 and  multiplying by 3.

You will notice that 12 divided by 5 happens to be 2.4.

Each graphics mode has a default aspect  ratio so that circles drawn will be

perfectly round. The reason you are  allowed to change the aspect ratio with

this function  is that you  may not  want  to draw a  perfectly round circle

every time. By  changing the aspect ratio values you  can draw a circle that

is  more elongated  either horizontally  or vertically.  This gives you much

more flexibility in drawing circular shapes.

Parameters:

XAspect - Horizontal portion of the aspect ratio

YAspect - Vertical portion of the aspect ratio

Examples:

BASIC

CALL SetAspect(12, 5)

PASCAL

SetAspect(12, 5);

********************************************************************************

SetBackColor(Color)

Sets the current background color.

Parameters:

Color - New background color

Examples:

BASIC

CALL SetBackColor(1)

PASCAL

SetBackColor(1);

********************************************************************************

SetCursor(Row, Column)

Sets  the  current  text  cursor  position  for  use  with the 'PrintString'

function.

Parameters:

Row - Text cursor row

Column - Text cursor column

Examples:

BASIC

CALL SetCursor(10, 1)

PASCAL

SetCursor(10, 1);

********************************************************************************

SetDisplayPage(DisplayPage)

Sets  the current  display page  which  will  be displayed  on the  computer

monitor. See the 'PageCopy' function for a description of video pages.

Parameters:

DisplayPage - Display page to set.

Examples:

BASIC

CALL SetDisplayPage(DisplayPage%)

PASCAL

SetDisplayPage(DisplayPage);

********************************************************************************

SetPalette(Palette, Color)

Allows you to redefine any one of the 16 palette registers.

Palettes registers are special areas in the video system that determine what

colors are displayed on the screen. There  are 16 in all allowing for the 16

different colors. By default each palette register has the same value as its

corresponding color number.  For example, palette register 1  has a value of

1, which is blue, and palette register 5 has a value of 5, which is magenta.

This means that all 16 palette registers initially have the values 0-15.

Parameters:

Palette - Palette register number to set

Color - New color value for palette register

Examples:

BASIC

CALL SetPalette(1, 10)

PASCAL

SetPalette(1, 10);

********************************************************************************

SetPlotColor(Color)

Sets  the current  drawing color  for use  in graphic  functions that do not

specify a color as one of the parameters.

Parameters:

Color - New drawing color value

Examples:

BASIC

CALL SetPlotColor(5)

PASCAL

SetPlotColor(5);

********************************************************************************

SetTextColor(Color)

Sets the current text color used by 'PrintString'.

Parameters:

Color - New text color value

Examples:

BASIC

CALL SetTextColor(14)

PASCAL

SetTextColor(14);

********************************************************************************

SetView(MinX, MinY, MaxX, MaxY)

Sets the screen viewport boundary values.

The  screen viewport  is an  area  in  which all  graphics functions  except

'PrintString' and 'PrintStringX'  must fit. If not, they  are clipped on the

viewport's boundaries. The  default size of the screen  viewport is the size

of the  screen mode being used.  For example, if you  were in the 320x200x16

color graphics mode  and drew a line from  0, 0, to 500, 199;  the line's X2

value would be clipped at 319 which is the default maximum horizontal border

value for that screen mode. You can set the screen viewport to any size that

you want. The only limitation is that the values can be no smaller or larger

than  the  physical  size  of  the  screen  mode  you  are  working  in. The

'PrintString' and  'PrintStringX' functions ignore  the screen viewport  and

place text anywhere  on the screen. This is because  you are working in text

screen coordinates and not graphics screen coordinates.

Parameters:

MinX - Minimum horizontal border value

MinY - Minimum vertical border value

MaxX - Maximum horizontal border value

MaxY - Maximum vertical border value

Examples:

BASIC

CALL SetView(10, 10, 309, 189)

PASCAL

SetView(10, 10, 309, 189);

********************************************************************************

SoundBufOff

Disables background processing of sounds.

See  the   'ExtSound'  function  for  a   description  of  background  sound

processing.

Parameters:

None

Examples:

BASIC

CALL SoundBufOff

PASCAL

SoundBufOff;

********************************************************************************

SoundBufOn

Enables background processing of sounds.

See  the  'ExtSound'  function  for   a  description  of  background  sound

processing.

Parameters:

None

Examples:

BASIC

CALL SoundBufOn

PASCAL

SoundOn;

********************************************************************************

SoundOff

Turns  off all  current sounds   off and  disables background  processing of

sounds.

See the 'ExtSound' function for a description of background sound processing.

Parameters:

None

Examples:

BASIC

CALL SoundOff

PASCAL

SoundOff;

********************************************************************************

SpeakerOff

Disables the exterior speaker.

Parameters:

None

Examples:

BASIC

CALL SpeakerOff

PASCAL

SpeakerOff;

********************************************************************************

SpeakerOn

Enables the exterior speaker.

Parameters:

None

Examples:

BASIC

CALL SpeakerOn

PASCAL

SpeakerOn;

********************************************************************************

WaitKey

Pauses the program and waits for a key to be pressed at the keyboard.

Parameters:

None

Examples:

BASIC

CALL WaitKey

PASCAL

WaitKey

********************************************************************************

XorCircle(X, Y, Radius)

XOR's a circle  centered at X, Y using the  specified radius, current values

of aspect ratio and current drawing color.

See the 'SetAspect' function for a description of the aspect ratio.

Parameters:

X - Horizontal center of circle

Y - Vertical center of circle

Radius - Radius of circle

Examples:

BASIC

CALL XorCircle(160, 100, 75)

PASCAL

XorCircle(160, 100, 75)

********************************************************************************

XorCircleC(X, Y, Radius, Color)

XOR's a circle  centered at X, Y using the  specified radius, current values

of aspect ratio and the specified color.

See the 'SetAspect' function for a complete description of the aspect ratio.

Parameters:

X - Horizontal center of circle

Y - Vertical center of circle

Radius - Radius of circle

Color - Color of circle

Examples:

BASIC

CALL XorCircleC(160, 100, 75, 1)

PASCAL

XorCircle(160, 100, 75, 1)

********************************************************************************

XorDrawBox(X1, Y1, X2, Y2, Color)

XOR's a line box from X1, Y1 to X2, Y2 using the specified color.

Parameters:

X1 - Upper left horizontal position of box

Y1 - Upper left vertical position of box

X2 - Lower right horizontal position of box

Y2 - Lower right vertical position of box

Color - Color of box

Examples:

BASIC

CALL XorDrawBox(0, 0, 319, 199, 1)

PASCAL

XorDrawBox(0, 0, 319, 199, 1);

********************************************************************************

XorDrawBoxC(X1, Y1, X2, Y2)

XOR's a line box from X1, Y1 to X2, Y2 using the current drawing color.

Parameters:

X1 - Upper left horizontal position of box

Y1 - Upper left vertical position of box

X2 - Lower right horizontal position of box

Y2 - Lower right vertical position of box

Examples:

BASIC

CALL XorDrawBoxC(10, 10, 100, 100)

PASCAL

XorDrawBoxC(10, 10, 100, 100);

********************************************************************************

XorEllipse(X, Y, MajorAxis, MinorAxis)

XOR's an ellipse centered at the specified X, Y position using the specified

major and minor axes and current drawing color.

See the 'Ellipse' function for a description of an ellipse.

Parameters:

X - Horizontal center of ellipse

Y - Vertical center of ellipse

MajorAxis - Size of major axis

MinorAxis - Size of minor axis

Examples:

BASIC

CALL XorEllipse(160, 100, 50, 40)

PASCAL

XorEllipse(160, 100, 50, 40);

********************************************************************************

XorEllipseC(X, Y, MajorAxis, MinorAxis, Color)

XOR's an ellipse centered at the specified X, Y position using the specified

major and minor axes and specified color.

See the 'Ellipse' function for a description of an ellipse.

Parameters:

X - Horizontal center of ellipse

Y - Vertical center of ellipse

MajorAxis - Size of major axis

MinorAxis - Size of minor axis

Color - Color of ellipse

Examples:

BASIC

CALL XorEllipseC(160, 100, 50, 40, 1)

PASCAL

XorEllipseC(160, 100, 50, 40, 1);

********************************************************************************

XorFillBox(X1, Y1, X2, Y2, Color)

XOR's a solid box from X1, Y1 to X2, Y2 using the specified color.

Parameters:

X1 - Upper left horizontal position of box

Y1 - Upper left vertical position of box

X2 - Lower right horizontal position of box

Y2 - Lower right vertical position of box

Color - Color of box

Examples:

BASIC

CALL XorFillBox(0, 0, 100, 75, 1)

PASCAL

XorFillBox(0, 0, 100, 75, 1);

********************************************************************************

XorFillBoxC(X1, Y1, X2, Y2)

XOR's a solid box from X1, Y1 to X2, Y2 using the current drawing color.

Parameters:

X1 - Upper left horizontal position of box

Y1 - Upper left vertical position of box

X2 - Lower right horizontal position of box

Y2 - Lower right vertical position of box

Examples:

BASIC

CALL XorFillBoxC(0, 0, 100, 75)

PASCAL

XorFillBoxC(0, 0, 100, 75);

********************************************************************************

XorLine(X1, Y1, X2, Y2)

XOR's a line from X1, Y1 to X2, Y2 using the current drawing color.

Parameters:

X1 - Upper left horizontal position of line

Y1 - Upper left vertical position of line

X2 - Lower right horizontal position of line

Y2 - Lower right vertical position of line

Examples:

BASIC

CALL XorLine(10, 15, 100, 150)

PASCAL

XorLine(10, 15, 100, 150);

********************************************************************************

XorLineC(X1, Y1, X2, Y2, Color)

XOR's a line from X1, Y1 to X2, Y2 using the specified color.

Parameters:

X1 - Upper left horizontal position of line

Y1 - Upper left vertical position of line

X2 - Lower right horizontal position of line

Y2 - Lower right vertical position of line

Color - Color of line

Examples:

BASIC

CALL XorLineC(10, 15, 100, 150, 1)

PASCAL

XorLineC(10, 15, 100, 150, 1);

********************************************************************************

XorLineRel(XRel, YRel)

XOR's a  line a relative distance  from the current X,  Y position using the

current drawing color.

You can give this function both positive and negative values.

Parameters:

XRel - Number points to draw line in a horizontal direction

YRel - Number points to draw line in a vertical direction

Examples:

BASIC

CALL XorLineRel(10, 10)

PASCAL

XorLineRel(10, 10);

********************************************************************************

XorLineRelC(XRel, YRel, Color)

XOR's a  line a relative distance  from the current X,  Y position using the

specified color.

You can give this function both positive and negative values.

Parameters:

XRel - Number points to draw line in a horizontal direction

YRel - Number points to draw line in a vertical direction

Color - Color of line

Examples:

BASIC

CALL XorLineRelC(-5, -10, 15)

PASCAL

XorLineRelC(-5, -10, 15);

********************************************************************************

XorLineTo(X, Y)

XOR's a line to  the specified X, Y position from the  current X, Y position

using the current drawing color.

Parameters:

X - Ending horizontal position of line

Y - Ending vertical position of line

Examples:

BASIC

CALL XorLineTo(50, 75)

PASCAL

XorLineTo(50, 75);

********************************************************************************

XorLineToC(X, Y, Color)

XOR's a line to  the specified X, Y position from the  current X, Y position

using the specified drawing color.

Parameters:

X - Ending horizontal position of line

Y - Ending vertical position of line

Color - Color of line

Examples:

BASIC

CALL XorLineToC(50, 75, 6)

PASCAL

XorLineToC(50, 75, 6);

********************************************************************************

XorPoint(X, Y)

XOR's  a point  at the  specified X,  Y position  using the  current drawing

color.

Parameters:

X - Horizontal position to plot point

Y - Vertical position to plot point

Examples:

BASIC

CALL XorPoint(160, 100)

PASCAL

XorPoint(160, 100);

********************************************************************************

XorPointC(X, Y, Color)

XOR's a point at the specified X, Y position using the specified color.

Parameters:

X - Horizontal position to plot point

Y - Vertical position to plot point

Color - Color of point

Examples:

BASIC

CALL XorPointC(160, 100, 5)

PASCAL

XorPointC(160, 100, 5);

********************************************************************************

QuickBASIC programming interface:

To use GRAFIX  with the QuickBASIC programming environment  simply type in the

following line:

QB /L GRAFQBS

This will  start QuickBASIC and load  in the interface for  GRAFIX which is in

the GRAFQBS.QLB quick library. The next  step to use GRAFIX in your QuickBASIC

programs  is  to  make  sure  that  every  program  you  write begins with the

following line:

'$INCLUDE: 'GRAFQBS.INC'

GRAFQBS.INC  contains  all  the  subprogram  definitions  necessary to use the

QuickBASIC  interface  for  GRAFIX.  The  interface  for  GRAFIX  expects  all

variables  passed to  it to  be of  type integer  so GRAFQBS.INC  contains the

following line:

DEFINT A-Z

This  makes sure  that all  variables used  in your  program are  of the  type

integer  by default.  If you  compile a  program from  outside the  QuickBASIC

programming environment, be  sure to specify the GRAFQBS.LIB  library when you

use the LINK program to create the EXE file.

The interface  provided for QuickBASIC  is for version  4.5. If you  are still

using version 4.0 following these steps to create a new set of libraries:

1) Compile the GRAFIX.QBS file with the BC.EXE program like this:

BC grafix.qbs;

2) Create a new object library with the LIB.EXE program like this:

LIB grafqbs -+grafix.obj,nul,grafqbs

3) Create a new quick library with the LINK.EXE program like this:

LINK /q grafqbs.lib,,nul,bqlb40

Turbo BASIC and PowerBASIC programming interfaces:

Using the the  Turbo BASIC and PowerBASIC interfaces for  GRAFIX are easy. For

Turbo BASIC make  sure that every program you write  starts with the following

line:

$INCLUDE "GRAFTBS.INC"

For  PowerBASIC  make  sure  that  every  program  you  write  starts with the

following line:

$INCLUDE "GRAFPBS.INC"

The interfaces for  Turbo BASIC and PowerBASIC are almost  entirely made up of

INLINE  assembler subprograms.  Be careful  when making  calls to GRAFIX using

both interfaces because no checking is done by either compiler to validate the

number of  parameters or type  of variables used  when you make  the call. For

example, if typed in  the following program all bets are off  as to what would

be drawn on the screen:

$INCLUDE "GRAFTBS.INC"

CALL MediumGraphics

X1 = 0

Y1 = 0

X2 = 319

Y2 = 199

Color = 1

CALL ExtLine(X1, Y1, X2, Y2, Color)

The ExtLine function only  expects 4 parameters to be passed to  it not 5. Pay

close attention to the number of  parameters a certain function requires. This

will save you  a lot of headache later. Another  problem you may encounter can

be found by just changing a few lines in the above example:

$INCLUDE "GRAFPBS.INC"

DEFDBL X

CALL MediumGraphics

X1 = 0

Y1 = 0

X2 = 319

Y2 = 199

CALL ExtLine(X1, Y1, X2, Y2)

The previous problem  of the number of parameters has  been fixed, but now the

call won't work because of the 'DEFDBL X' statement. This causes all variables

starting with  the letter X to  be of type double  precision. Double precision

variables are floating point numbers 8 bytes in length each. The interface for

GRAFIX  expects all  variables passed  to it  to be  of type  integer. Integer

variables  are whole  numbers 2  bytes  in  length each.  The GRAFTBS.INC  and

GRAFPBS.INC files contains the following line:

DEFINT A-Z

This  makes sure  that all  variables used  in your  program are  of the  type

integer by default.

Also be careful  when using an implicit variable type  other than integer. The

following example shows what I mean:

$INCLUDE "GRAFPBS.INC"

CALL MediumGraphics

X1# = 0

Y1# = 0

X2# = 319

Y2# = 199

CALL ExtLine(X1#, Y1#, X2#, Y2#)

When you attach  '#' to a variable you  are declaring it to be  of type double

precision. Just be careful the type of  variable you pass to a GRAFIX function

since no type checking is done by the either compiler.

QuickPASCAL and Turbo PASCAL programming interface:

The interfaces for  QuickPASCAL and Turbo PASCAL are identical  in the way you

incorporate them into your program. To  use GRAFIX in your PASCAL program just

list GRAFIX as one of the units in your program's USES statement like this:

USES

Grafix;

That's all there is to it.

The compiled GRAFIX unit for Turbo PASCAL and QuickPASCAL is not provided with

the package. The is due to the fact that compiled units for different versions

of Turbo PASCAL are not compatible. The same may hold true for future versions

of QuickPASCAL as  well. You can easily create the  GRAFIX unit by loading the

program GRAFIX.PAS into either compiler and compiling it to disk.

CHAPTER 3  - ASSEMBLER Programming Interface

This chapter gives the technical details of the GRAFIX package. It will give you

the  necessary  information  to  write  an  ASSEMBLY  language interface for the

programming language you  are working with. I'm going to  assume that anyone who

reads this chapter  already knows how to program in  80x86 ASSEMBLY language. If

you don't  then I suggest that  you skip this section.  It probably will confuse

you more than help.

An ASSEMBLY language interface for GRAFIX is  the key to writing an interface in

another programming language. The BASIC  and PASCAL interfaces provided with the

package  are written  in ASSEMBLER.  GRAFIX is  a memory  resident program,  all

requests  to it  are done  through  the  video BIOS  interrupt. The  AH register

contains  the hexadecimal  value EE  which indicates  to GRAFIX  that a function

request is about  to be made. The AL register  contains the function number. The

remaining registers  BX, CX, DX,  SI, DI, and  DS contain the  parameters of the

requested function.  For example, if  you wanted  to  draw a solid  green box on

screen from 0, 0 - 319, 199 the function call would look like this:

MOV     AX,0EE11H       ;Draw solid box function

MOV     BX,0            ;X1 location in BX

MOV     CX,0            ;Y1 location in CX

MOV     SI,319          ;X2 location in SI

MOV     DI,199          ;Y2 location in DI

MOV     DX,2            ;Box color in DX

INT     10H             ;Call GRAFIX

The  next section  of this  chapter will  list each  of the  GRAFIX functions in

numerical order. The format of each function listing is as follows:

Title

Function number

Description

Register entry values

Register exit values

Example

Comments

The title  is the name  I use for  the function in  the ASSEMBLY language source

file, it is just listed here for  reference purposes. The function number is the

the value for  the AL register, it is listed  in decimal format. The description

explains  what each  function does.  The register  entry values  are the  values

required to execute  the function. The exit values are  any register values that

the function might  return to your program. The example  is an ASSEMBLY language

code fragment showing  a call to GRAFIX for that  function. The comments section

is optional  and is included  only where I  feel extra clarification  is needed.

Entry values for the AX register for each function are in hexadecimal.

********************************************************************************

TITLE:

ClearScreen

FUNCTION NUMBER:

00

DESCRIPTION:

Clears the screen viewport to the current background color.

ENTRY VALUES:

AX = EE00

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE00H       ;Clear screen function

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'SetView'  function in chapter 2 for a  more information of the screen

viewport.

********************************************************************************

TITLE:

ColorEnable

FUNCTION NUMBER:

01

DESCRIPTION:

Turns the color enable signal on or off.

ENTRY VALUES:

AX = EE01

BX = Action

0 = Turn color enable signal off

1 = Turn color enable signal on

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE01H       ;Color enable signal function

MOV     BX,0            ;Turn color enable signal off

INT     10H             ;Call GRAFIX

COMMENTS:

This  function  is  mainly  provided  for  people  with  monochrome  composite

monitors.  The color  graphics are  much easier  to see  when the color enable

signal is turned off.

********************************************************************************

TITLE:

SetColor

FUNCTION NUMBER:

02

DESCRIPTION:

Sets the current drawing, text, or background color.

ENTRY VALUES:

AX = EE02

BX = Color type

01 = Drawing color

02 = Text color

03 = Background color

CX = Color

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE02H       ;Set color function

MOV     BX,1            ;Set drawing color

MOV     CX,4            ;Set drawing color to RED

INT     10H             ;Call GRAFIX

COMMENTS:

The drawing  color is the color  that will be used  for all graphics functions

that do not  specify a color as one  of the parameters. The text  color is the

color of the characters printed  with the PrintString function. The background

color is the color of the screen background and border.

********************************************************************************

TITLE:

GetColor

FUNCTION NUMBER:

03

DESCRIPTION:

Returns the current drawing, text, or background color.

ENTRY VALUES:

AX = EE03

BX = Color type

01 = Return drawing color

02 = Return text color

03 = Return background color

EXIT VALUES:

CX = Color

EXAMPLE:

MOV     AX,0EE03H       ;Get color function

MOV     BX,3            ;Return current background color

INT     10H             ;Call GRAFIX

MOV     [BColor],CX     ;Store current background color

COMMENTS:

None

********************************************************************************

TITLE:

SetAspect

FUNCTION NUMBER:

04

DESCRIPTION:

Sets the  vertical and horizontal  elements of the  aspect ratio used  to draw

circles.

ENTRY VALUES:

AX = EE04

BX = Horizontal portion of aspect ratio

CX = Vertical portion of aspect ratio

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE04H       ;Set aspect ratio function

MOV     BX,12           ;Horizontal portion of aspect ratio

MOV     CX,5            ;Vertical portion of aspect ratio

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'SetAspect' function in chapter 2 for more information.

********************************************************************************

TITLE:

GetAspect

FUNCTION NUMBER:

05

DESCRIPTION:

Returns the current horizontal and vertical portions of the aspect ratio.

ENTRY VALUES:

AX = EE05

EXIT VALUES:

BX = Horizontal portion of aspect ratio

CX = Vertical portion of aspect ratio

EXAMPLE:

MOV     AX,0EE05H       ;Get aspect ratio function

INT     10H             ;Call GRAFIX

MOV     [HAspect],BX    ;Store horizontal portion of aspect ration

MOV     [VAspect],CX    ;Store vertical portion of aspect ration

COMMENTS:

None

********************************************************************************

TITLE:

Point

FUNCTION NUMBER:

06

DESCRIPTION:

Plots individual points onto the screen using the current drawing color.

ENTRY VALUES:

AX = EE06

SI = X position

DI = Y position

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE06H       ;Plot point function

MOV     SI,160          ;X position

MOV     DI,100          ;Y position

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

GetPoint

FUNCTION NUMBER:

07

DESCRIPTION:

Returns the current color of the point at the specified X, Y position.

ENTRY VALUES:

AX = EE07

SI = X position

DI = Y position

EXIT VALUES:

AX = Color of point

EXAMPLE:

MOV     AX,0EE07H       ;Get color of point function

MOV     SI,160          ;X position

MOV     DI,100          ;Y position

INT     10H             ;Call GRAFIX

MOV     [Point],AX      ;Store color of point

COMMENTS:

None

********************************************************************************

TITLE:

MoveTo

FUNCTION NUMBER:

08

DESCRIPTION:

Updates the current X, Y position by replacing the previous X, Y values.

ENTRY VALUES:

AX = EE08

SI = New X position

DI = New Y position

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE08H       ;Move to function

MOV     SI,0            ;X position

MOV     DI,0            ;Y position

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

MoveRel

FUNCTION NUMBER:

09

DESCRIPTION:

Updates the current X, Y position by adding to the previous X, Y values.

ENTRY VALUES:

AX = EE09

SI = Number of points to add horizontally

DI = Number of points to add vertically

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE09H       ;Move relative function

MOV     SI,10           ;Horizontal points to move

MOV     DI,10           ;Vertical points to move

INT     10H             ;Call GRAFIX

COMMENTS:

You can give this function either  positive or negative values allowing you to

move up, down, left, and right.

********************************************************************************

TITLE:

GetXY

FUNCTION NUMBER:

10

DESCRIPTION:

Returns the current X, Y position.

ENTRY VALUES:

AX = EE0A

EXIT VALUES:

BX = Current X position

CX = Current Y position

EXAMPLE:

MOV     AX,0EE0AH       ;Get current X, Y function

INT     10H             ;Call GRAFIX

MOV     [X],BX          ;Store current X position

MOV     [Y],CX          ;Store current Y position

COMMENTS:

None

********************************************************************************

TITLE:

Line

FUNCTION NUMBER:

11

DESCRIPTION:

Draws a line starting at the upper left  X, Y position to the lower right X, Y

position using the current drawing color.

ENTRY VALUES:

AX = EE0B

BX = Upper left X position

CX = Upper left Y position

SI = Lower right X position

DI = Lower right Y position

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE0BH       ;Draw line function

MOV     BX,0            ;Upper left X

MOV     CX,0            ;Upper left Y

MOV     SI,319          ;Lower right X

MOV     DI,199          ;Lower right Y

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

LineTo

FUNCTION NUMBER:

12

DESCRIPTION:

Draws a line from  the current X, Y position to specified  X, Y position using

the current drawing color.

ENTRY VALUES:

AX = EE0C

SI = Ending X position

DI = Ending Y position

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE0CH       ;Draw line to function

MOV     SI,319          ;Ending X

MOV     DI,199          ;Ending Y

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

LineRel

FUNCTION NUMBER:

13

DESCRIPTION:

Draws  a line  from the  current X,  Y position  adding the  number of  points

horizontally and vertically to get the ending X, Y position. The line is drawn

using the current drawing color.

ENTRY VALUES:

AX = EE0D

SI = Number of points to add horizontally

DI = Number of points to add vertically

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE0DH       ;Draw line relative function

MOV     SI,-10          ;Number of points to add horizontally

MOV     DI,-10          ;Number of points to add vertically

INT     10H             ;Call GRAFIX

COMMENTS:

You can give this function both positive and negative values.

********************************************************************************

TITLE:

DrawBox

FUNCTION NUMBER:

14

DESCRIPTION:

Draws a line box  starting at the upper left X, Y  position to the lower right

X, Y position using the specified color.

ENTRY VALUES:

AX = EE0E

BX = Upper left X position

CX = Upper left Y position

SI = Lower right X position

DI = Lower right Y position

DX = Color of box

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE0EH       ;Draw box with color function

MOV     BX,0            ;Upper left X

MOV     CX,0            ;Upper left Y

MOV     SI,100          ;Lower right X

MOV     DI,100          ;Lower right Y

MOV     DX,1            ;Box color

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

Circle

FUNCTION NUMBER:

15

DESCRIPTION:

Draws a  circle centered at  the specified X,  Y position using  the specified

radius, and current drawing color.

ENTRY VALUES:

AX = EE0F

SI = X origin

DI = Y origin

BX = Radius

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE0FH       ;Draw circle function

MOV     SI,160          ;X origin

MOV     DI,100          ;Y origin

MOV     BX,50           ;Radius

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'SetApect' function in chapter 2 for description of the aspect ratio.

********************************************************************************

TITLE:

Paint

FUNCTION NUMBER:

16

DESCRIPTION:

Paints  irregular shapes  starting at  the specified  X, Y  position using the

specified fill color stopping at the specified boundary color.

ENTRY VALUES:

AX = EE10

SI = Starting X position

DI = Starting Y position

BX = Fill color

CX = Boundary color

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE10H       ;Paint function

MOV     SI,100          ;X starting position

MOV     DI,100          ;Y starting position

MOV     BX,9            ;Fill color

MOV     CX,14           ;Boundary color

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'ExtPaint' function in chapter 2 for a detailed explanation.

********************************************************************************

TITLE:

FillBox

FUNCTION NUMBER:

17

DESCRIPTION:

Draws a solid box starting at the upper  left X, Y position to the lower right

X, Y position using the specified color.

ENTRY VALUES:

AX = EE11

BX = Upper left X position

CX = Upper left Y position

SI = Lower right X position

DI = Lower right Y position

DX = Color of box

EXAMPLE:

MOV     AX,0EE11H       ;Fill box with color function

MOV     BX,0            ;X1 position

MOV     CX,0            ;Y1 position

MOV     SI,100          ;X2 position

MOV     DI,100          ;Y2 position

MOV     DX,5            ;Box color

INT     10H             ;Call GRAFIX

EXIT VALUES:

None

COMMENTS:

None

********************************************************************************

TITLE:

SetPalette

FUNCTION NUMBER:

18

DESCRIPTION:

Sets the specified palette register to the specified color.

ENTRY VALUES:

AX = EE12

BX = Palette register number

CX = Color value to store

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE12H       ;Set palette register function

MOV     BX,1            ;Palette register number

MOV     CX,0            ;Color to store

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'SetPalette' function in chapter 2 for more information.

********************************************************************************

TITLE:

PaletteUsing

FUNCTION NUMBER:

19

DESCRIPTION:

Sets all 16 palette registers using 16 elements of an integer array.

ENTRY VALUES:

AX = EE13

DS = Segment address of integer array

DX = Offset address of integer array

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE13H       ;Set all 16 palette registers function

MOV     DX,SEG Array    ;Segment address of array

MOV     DS,DX           ; ..

MOV     DX,OFFSET Array ;Offset address of array

INT     10H             ;Call GRAFIX

COMMENTS:

See  the 'SetPalette'  function in  chapter 2  for a  detailed explanation  of

palette registers.

********************************************************************************

TITLE:

ResetPalette

FUNCTION NUMBER:

20

DESCRIPTION:

Resets all 16 palette registers to their default state.

ENTRY VALUES:

AX = EE14

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE14H       ;Reset palette registers function

INT     10H             ;Call GRAFIX

COMMENTS:

See  the 'SetPalette'  function in  chapter 2  for a  detailed explanation  of

palette registers.

********************************************************************************

TITLE:

Get

FUNCTION NUMBER:

21

DESCRIPTION:

Stores a graphics image starting at the  upper left X, Y position to the lower

right X, Y position into an integer array.

ENTRY VALUES:

AX = EE15

BX = Upper left X position

CX = Upper left Y position

SI = Lower right X position

DI = Lower right Y position

DS = Segment address of integer array

DX = Offset address of integer array

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE15H       ;Store graphics image function

MOV     BX,0            ;X1 position

MOV     CX,0            ;Y1 position

MOV     SI,50           ;X2 position

MOV     DI,75           ;Y2 position

MOV     DX,SEG Array    ;Segment address of array

MOV     DS,DX           ; ..

MOV     DX,OFFSET Array ;Offset address of array

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'ExtGet' function in chapter 2 for more information.

********************************************************************************

TITLE:

Put

FUNCTION NUMBER:

22

DESCRIPTION:

Transfers an image  that was previously stored using the  Get function using 1

of 5 actions.

ENTRY VALUES:

AX = EE16

SI = Starting X position

DI = Starting Y position

BX = Action

1 = PRESET

2 = PSET

3 = AND

4 = OR

5 = XOR

DS = Segment address of integer array

DX = Offset address of integer array

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE16H       ;Transfer graphics image function

MOV     SI,10           ;Starting X position

MOV     DI,30           ;Starting Y position

MOV     BX,5            ;Use XOR action

MOV     DX,SEG Array    ;Segment address of array

MOV     DS,DX           ; ..

MOV     DX,OFFSET Array ;Offset address of array

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'ExtPut' function in chapter 2 for more information.

********************************************************************************

TITLE:

SetCursor

FUNCTION NUMBER:

23

DESCRIPTION:

Turns the graphics cursor on/off or sets it to full/half size.

ENTRY VALUES:

AX = EE17

BX = Action

0 = Turn graphics cursor off

1 = Turn graphics cursor on

2 = Set cursor to full size

3 = Set cursor to half size

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE17H       ;Set graphics cursor function

MOV     BX,0            ;Turn cursor off

INT     10H             ;Call GRAFIX

COMMENTS:

The reason  I provided a  cursor was for  the text support.  When you turn the

cursor on  it acts exactly  like the cursor  you have in  text screen mode. It

advances 8 points at time up and down so it will always cover a text character

evenly.  One example  of using  the text  cursor is  if you  develop an  input

routine. You could  turn the cursor on to show  the current position where the

person was typing.  You could use the half cursor  size to indicate the person

had pressed the insert key and now the insert state is active.

********************************************************************************

TITLE:

GetCursor

FUNCTION NUMBER:

24

DESCRIPTION:

Returns the current graphics cursor status and size.

ENTRY VALUES:

AX = EE18

EXIT VALUES:

BX = Cursor status

0 = Graphics cursor off

1 = Graphics cursor on

CX = Cursor size

0 = Cursor is full size

1 = Cursor is half size

EXAMPLE:

MOV     AX,0EE18H       ;Get graphics cursor status/size function

INT     10H             ;Call GRAFIX

MOV     [Status],BX     ;Store cursor status

MOV     [CursorSize],CX ;Store cursor size

COMMENTS:

None

********************************************************************************

TITLE:

SetCursorPos

FUNCTION NUMBER:

25

DESCRIPTION:

Sets  the current  graphics cursor  position in  text format  for use with the

'PrintString' function.

ENTRY VALUES:

AX = EE19

BX = Column

CX = Row

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE19H       ;Set graphics cursor position function

MOV     BX,10           ;Column

MOV     CX,15           ;Row

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

GetCursorPos

FUNCTION NUMBER:

26

DESCRIPTION:

Returns the current graphics cursor position in text format.

ENTRY VALUES:

AX = EE1A

EXIT VALUES:

BX = Current column

CX = Current row

EXAMPLE:

MOV     AX,0EE1AH       ;Get graphics cursor position function

INT     10H             ;Call GRAFIX

MOV     [Column],BX     ;Store column

MOV     [Row],CX        ;Store row

COMMENTS:

None

********************************************************************************

TITLE:

PrintString

FUNCTION NUMBER:

27

DESCRIPTION:

Prints a  text string on  the graphics screen  starting at the  current cursor

position using the current text color.

ENTRY VALUES:

AX = EE1B

BX = Action when printing string

0 = Don't advance to the next line after printing the string

1 = Advance to the next line after printing the string

CX = Length of string

DS = Segment address of string

DX = Offset address of string

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE1BH       ;Print text string function

MOV     BX,1            ;Use an ending carriage return/line feed

MOV     CX,10           ;Length of string

MOV     DX,SEG String   ;Segment address of string

MOV     DS,DX           ; ..

MOV     DX,OFFSET String;Offset address of string

INT     10H             ;Call GRAFIX

COMMENTS:

Using the 'SetColor'  function you can use any  of the 16 colors to  print the

string with. Using  the 'SetCursor' function you control  the starting row and

column location the string will be printed at.

********************************************************************************

TITLE:

Animate

FUNCTION NUMBER:

28

DESCRIPTION:

Animates an image  doing both the initial transfer and  final erase all in one

step. The  delay is the amount  of time to pause  between the initial transfer

and final erase of the image. Use this to minimize flickering.

ENTRY VALUES:

AX = EE1C

SI = Starting X position

DI = Starting Y position

DS = Segment address of array

DX = Offset address of array

CX = Delay (1 - 65535)

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE1CH       ;Animate function

MOV     SI,50           ;Starting X position

MOV     DI,75           ;Starting Y position

MOV     CX,100          ;Amount of delay

MOV     DX,SEG Array    ;Segment address of array

MOV     DS,DX           ; ..

MOV     DX,OFFSET Array ;Offset address of array

INT     10H             ;Call GRAFIX

COMMENTS:

See  the  'Animate'  and  'ExtPut'  functions  in  chapter  2  for  a detailed

explanation.

********************************************************************************

TITLE:

PointC

FUNCTION NUMBER:

29

DESCRIPTION:

Plots individual points onto the screen using the specified color.

ENTRY VALUES:

AX = EE1D

SI = X position

DI = Y position

DX = Color of point

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE1DH       ;Plot point with color function

MOV     SI,10           ;X position

MOV     DI,10           ;Y position

MOV     DX,3            ;Color of point

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

LineC

FUNCTION NUMBER:

30

DESCRIPTION:

Draws a line starting at the upper left  X, Y position to the lower right X, Y

position using the specified color.

ENTRY VALUES:

AX = EE1E

BX = Upper left X position

CX = Upper left Y position

SI = Lower right X position

DI = Lower right Y position

DX = Color of line

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE1EH       ;Draw line with color function

MOV     BX,10           ;X1 position

MOV     CX,10           ;Y1 position

MOV     SI,150          ;X2 position

MOV     DI,199          ;Y2 position

MOV     DX,1            ;Color of line

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

LineToC

FUNCTION NUMBER:

31

DESCRIPTION:

Draws a line starting at the current  X, Y position to specified X, Y position

using the specified color.

ENTRY VALUES:

AX = EE1F

SI = X position to line to

DI = Y position to line to

DX = Color of line

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE1FH       ;Draw line to with color function

MOV     SI,150          ;Ending X position

MOV     DI,199          ;Ending Y position

MOV     DX,10           ;Color of line

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

LineRelC

FUNCTION NUMBER:

32

DESCRIPTION:

Draws  a line  from the  current X,  Y position  adding the  number of  points

horizontally and vertically to get the ending X, Y position. The line is drawn

using the specified color.

ENTRY VALUES:

AX = EE20

SI = Number of points to move horizontally

DI = Number of points to move vertically

DX = Color of line

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE20H       ;Draw line relative with color function

MOV     SI,5            ;Number of points to move horizontally

MOV     DI,10           ;Number of points to move vertically

MOV     DX,2            ;Color of line

INT     10H             ;Call GRAFIX

COMMENTS:

You can give this function both positive and negative values.

********************************************************************************

TITLE:

CircleC

FUNCTION NUMBER:

33

DESCRIPTION:

Draws a  circle centered at  the specified X,  Y position using  the specified

radius, color.

ENTRY VALUES:

AX = EE21

SI = Starting X position

DI = Starting Y position

BX = Radius

DX = Color of circle

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE21H       ;Draw circle with color function

MOV     SI,160          ;X origin

MOV     DI,10           ;Y origin

MOV     BX,50           ;Radius

MOV     DX,15           ;Color of circle

INT     10H             ;Call GRAFIX

COMMENTS:

See the  'SetAspect' function in chapter  2 for a detailed  explanation of the

aspect ratio.

********************************************************************************

TITLE:

DrawBoxC

FUNCTION NUMBER:

34

DESCRIPTION:

Draws a line box  starting at the upper left X, Y  position to the lower right

X, Y position using the current drawing color.

ENTRY VALUES:

AX = EE22

BX = Upper left X position

CX = Upper left Y position

SI = Lower right X position

DI = Lower right Y position

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE22H       ;Draw box function

MOV     BX,0            ;X1 position

MOV     CX,0            ;X2 position

MOV     SI,319          ;X2 position

MOV     DI,199          ;Y2 position

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

FillBoxC

FUNCTION NUMBER:

35

DESCRIPTION:

Draws a solid box starting at the upper  left X, Y position to the lower right

X, Y using the current drawing color.

ENTRY VALUES:

AX = EE23

BX = Upper left X position

CX = Upper left Y position

SI = Lower right X position

DI = Lower right Y position

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE23H       ;Fill box function

MOV     BX,0            ;X1 position

MOV     CX,0            ;X2 position

MOV     SI,319          ;X2 position

MOV     DI,199          ;Y2 position

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

XorPoint

FUNCTION NUMBER:

36

DESCRIPTION:

XOR's individual points onto the screen using the current drawing color.

ENTRY VALUES:

AX = EE24

SI = X position

DI = Y position

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE24H       ;Xor point function

MOV     SI,160          ;X position

MOV     DI,100          ;Y position

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

XorPointC

FUNCTION NUMBER:

37

DESCRIPTION:

XOR's individual points onto the screen using the specified color.

ENTRY VALUES:

AX = EE25

SI = X position

DI = Y position

DX = Color of point

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE25H       ;Xor point with color function

MOV     SI,160          ;X position

MOV     DI,100          ;Y position

MOV     DX,1            ;Point color

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

XorLine

FUNCTION NUMBER:

38

DESCRIPTION:

XOR's a line starting at the upper left  X, Y position to the lower right X, Y

position using the current drawing color.

ENTRY VALUES:

AX = EE26

BX = Upper left X position

CX = Upper left Y position

SI = Lower right X position

DI = Lower right Y position

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE26H       ;Xor line function

MOV     BX,0            ;Upper left X

MOV     CX,0            ;Upper left Y

MOV     SI,319          ;Lower right X

MOV     DI,199          ;Lower right Y

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

XorLineC

FUNCTION NUMBER:

39

DESCRIPTION:

XOR's a line starting at the upper left  X, Y position to the lower right X, Y

position using the specified color.

ENTRY VALUES:

AX = EE27

BX = Upper left X position

CX = Upper left Y position

SI = Lower right X position

DI = Lower right Y position

DX = Color of line

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE27H       ;Xor line with color function

MOV     BX,0            ;Upper left X

MOV     CX,0            ;Upper left Y

MOV     SI,319          ;Lower right X

MOV     DI,199          ;Lower right Y

MOV     DX,12           ;Line color

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

XorLineTo

FUNCTION NUMBER:

40

DESCRIPTION:

XOR's a line from  the current X, Y position to specified  X, Y position using

the current drawing color.

ENTRY VALUES:

AX = EE28

SI = Ending X position

DI = Ending Y position

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE28H       ;Xor line to function

MOV     SI,319          ;Ending X

MOV     DI,199          ;Ending Y

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

XorLineToC

FUNCTION NUMBER:

41

DESCRIPTION:

XOR's a line from  the current X, Y position to specified  X, Y position using

the specified color.

ENTRY VALUES:

AX = EE29

SI = Ending X position

DI = Ending Y position

DX = Line color

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE29H       ;Xor line to with color function

MOV     SI,319          ;Ending X

MOV     DI,199          ;Ending Y

MOV     DX,5            ;Line color

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

XorLineRel

FUNCTION NUMBER:

42

DESCRIPTION:

XOR's  a line  from the  current X,  Y position  adding the  number of  points

horizontally and vertically to get the ending X, Y position. The line is drawn

using the current drawing color.

ENTRY VALUES:

AX = EE2A

SI = Number of points to add horizontally

DI = Number of points to add vertically

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE2AH       ;Xor line relative function

MOV     SI,10           ;Number of points to add horizontally

MOV     DI,10           ;Number of points to add vertically

INT     10H             ;Call GRAFIX

COMMENTS:

You can give this function both positive and negative values.

********************************************************************************

TITLE:

XorLineRelC

FUNCTION NUMBER:

43

DESCRIPTION:

XOR's  a line  from the  current X,  Y position  adding the  number of  points

horizontally and vertically to get the ending X, Y position. The line is drawn

using the specified color.

ENTRY VALUES:

AX = EE2B

SI = Number of points to add horizontally

DI = Number of points to add vertically

DX = Line color

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE2BH       ;Xor line relative with color function

MOV     SI,-10          ;Number of points to add horizontally

MOV     DI,-10          ;Number of points to add vertically

MOV     DX,3            ;Line color

INT     10H             ;Call GRAFIX

COMMENTS:

You can give this function both positive and negative values.

********************************************************************************

TITLE:

XorDrawBox

FUNCTION NUMBER:

44

DESCRIPTION:

XOR's a line box  starting at the upper left X, Y  position to the lower right

X, Y position using the specified color.

ENTRY VALUES:

AX = EE2C

BX = Upper left X position

CX = Upper left Y position

SI = Lower right X position

DI = Lower right Y position

DX = Color of box

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE2CH       ;Xor box with color function

MOV     BX,0            ;Upper left X

MOV     CX,0            ;Upper left Y

MOV     SI,100          ;Lower right X

MOV     DI,100          ;Lower right Y

MOV     DX,1            ;Box color

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

XorDrawBoxC

FUNCTION NUMBER:

45

DESCRIPTION:

XOR's a line box  starting at the upper left X, Y  position to the lower right

X, Y position using the current drawing color.

ENTRY VALUES:

AX = EE2D

BX = Upper left X position

CX = Upper left Y position

SI = Lower right X position

DI = Lower right Y position

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE2DH       ;Xor box function

MOV     BX,0            ;Upper left X

MOV     CX,0            ;Upper left Y

MOV     SI,100          ;Lower right X

MOV     DI,100          ;Lower right Y

INT     10H             ;Call GRAFIX

COMMENTS:

None

********************************************************************************

TITLE:

XorFillBox

FUNCTION NUMBER:

46

DESCRIPTION:

XOR's a solid box starting at the upper  left X, Y position to the lower right

X, Y position using the specified color.

ENTRY VALUES:

AX = EE2E

BX = Upper left X position

CX = Upper left Y position

SI = Lower right X position

DI = Lower right Y position

DX = Color of box

EXAMPLE:

MOV     AX,0EE2EH       ;Xor fill box with color function

MOV     BX,0            ;X1 position

MOV     CX,0            ;Y1 position

MOV     SI,100          ;X2 position

MOV     DI,100          ;Y2 position

MOV     DX,5            ;Box color

INT     10H             ;Call GRAFIX

EXIT VALUES:

None

COMMENTS:

None

********************************************************************************

TITLE:

XorFillBoxC

FUNCTION NUMBER:

47

DESCRIPTION:

XOR's a solid box starting at the upper  left X, Y position to the lower right

X, Y position using the current drawing color.

ENTRY VALUES:

AX = EE2F

BX = Upper left X position

CX = Upper left Y position

SI = Lower right X position

DI = Lower right Y position

EXAMPLE:

MOV     AX,0EE2FH       ;Xor fill box function

MOV     BX,0            ;X1 position

MOV     CX,0            ;Y1 position

MOV     SI,100          ;X2 position

MOV     DI,100          ;Y2 position

INT     10H             ;Call GRAFIX

EXIT VALUES:

None

COMMENTS:

None

********************************************************************************

TITLE:

XorCircle

FUNCTION NUMBER:

48

DESCRIPTION:

XOR's a  circle centered at  the specified X,  Y position using  the specified

radius, and current drawing color.

ENTRY VALUES:

AX = EE30

SI = X origin

DI = Y origin

BX = Radius

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE30H       ;Xor circle function

MOV     SI,160          ;X origin

MOV     DI,100          ;Y origin

MOV     BX,50           ;Radius

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'SetApect' function in chapter 2 for description of the aspect ratio.

********************************************************************************

TITLE:

XorCircleC

FUNCTION NUMBER:

49

DESCRIPTION:

XOR's a  circle centered at  the specified X,  Y position using  the specified

radius, and color.

ENTRY VALUES:

AX = EE31

SI = X origin

DI = Y origin

BX = Radius

DX = Circle color

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE31H       ;Xor circle with color function

MOV     SI,160          ;X origin

MOV     DI,100          ;Y origin

MOV     BX,50           ;Radius

MOV     DX,8            ;Circle color

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'SetApect' function in chapter 2 for description of the aspect ratio.

********************************************************************************

TITLE:

Ellipse

FUNCTION NUMBER:

50

DESCRIPTION:

Draws an ellipse  centered at the specified X, Y  position using the specified

major and minor axes and current drawing color.

ENTRY VALUES:

AX = EE32

SI = X origin

DI = Y origin

BX = Major axis

CX = Minor axis

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE32H       ;Ellipse function

MOV     SI,160          ;X origin

MOV     DI,100          ;Y origin

MOV     BX,50           ;Major axis

MOV     CX,75           ;Minor axis

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'Ellipse' function in chapter 2 for more information.

********************************************************************************

TITLE:

EllipseC

FUNCTION NUMBER:

51

DESCRIPTION:

Draws an ellipse  centered at the specified X, Y  position using the specified

major and minor axes and color.

ENTRY VALUES:

AX = EE33

SI = X origin

DI = Y origin

BX = Major axis

CX = Minor axis

DX = Ellipse color

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE33H       ;Ellipse with color function

MOV     SI,160          ;X origin

MOV     DI,100          ;Y origin

MOV     BX,50           ;Major axis

MOV     CX,75           ;Minor axis

MOV     DX,7            ;Ellipse color

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'Ellipse' function in chapter 2 for more information.

********************************************************************************

TITLE:

XorEllipse

FUNCTION NUMBER:

52

DESCRIPTION:

XOR's an ellipse  centered at the specified X, Y  position using the specified

major and minor axes and current drawing color.

ENTRY VALUES:

AX = EE34

SI = X origin

DI = Y origin

BX = Major axis

CX = Minor axis

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE34H       ;Xor ellipse function

MOV     SI,160          ;X origin

MOV     DI,100          ;Y origin

MOV     BX,50           ;Major axis

MOV     CX,75           ;Minor axis

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'Ellipse' function in chapter 2 for more information.

********************************************************************************

TITLE:

XorEllipseC

FUNCTION NUMBER:

53

DESCRIPTION:

XOR's an ellipse  centered at the specified X, Y  position using the specified

major and minor axes and color.

ENTRY VALUES:

AX = EE35

SI = X origin

DI = Y origin

BX = Major axis

CX = Minor axis

DX = Ellipse color

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE35H       ;Xor ellipse with color function

MOV     SI,160          ;X origin

MOV     DI,100          ;Y origin

MOV     BX,50           ;Major axis

MOV     CX,75           ;Minor axis

MOV     DX,7            ;Ellipse color

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'Ellipse' function in chapter 2 for more information.

********************************************************************************

TITLE:

SetView

FUNCTION NUMBER:

54

DESCRIPTION:

Sets the size of the current screen viewport.

ENTRY VALUES:

AX = EE36

BX = Minimum X

CX = Minimum Y

SI = Maximum X

DI = Maximum Y

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE36H       ;Set screen viewport function

MOV     BX,10           ;Minimum X

MOV     CX,10           ;Minimum Y

MOV     SI,309          ;Maximum X

MOV     DI,189          ;Maximum Y

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'SetView' function in chapter 2 for more information.

********************************************************************************

TITLE:

GetView

FUNCTION NUMBER:

55

DESCRIPTION:

Returns the size of the current screen viewport.

ENTRY VALUES:

AX = EE37

EXIT VALUES:

BX = Minimum X

CX = Minimum Y

SI = Maximum X

DI = Maximum Y

EXAMPLE:

MOV     AX,0EE37H       ;Get screen viewport function

INT     10H             ;Call GRAFIX

MOV     [MinX],BX       ;Store minimum X

MOV     [MinY],CX       ;Store minimum Y

MOV     [MaxX],SI       ;Store maximum X

MOV     [MaxY],DI       ;Store maximum Y

COMMENTS:

See the 'SetView' function in chapter 2 for more information.

********************************************************************************

TITLE:

ResetView

FUNCTION NUMBER:

56

DESCRIPTION:

Resets the screen viewport to it's default size.

ENTRY VALUES:

AX = EE38

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE38H       ;Reset screen viewport function

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'SetView' function in chapter 2 for more information.

********************************************************************************

TITLE:

GetScreen

FUNCTION NUMBER:

57

DESCRIPTION:

Copies the entire screen buffer into an integer array.

ENTRY VALUES:

AX = EE39

DS = Segment address of integer array

DX = Offset address of integer array

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE39H       ;Copy screen function

MOV     DX,SEG Array    ;Get segment address of array

MOV     DS,DX           ; ..

MOV     DX,OFFSET Array ;Get offset address of array

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'GetScreen' function in chapter 2 for more information.

********************************************************************************

TITLE:

PutScreen

FUNCTION NUMBER:

58

DESCRIPTION:

Copies the entire screen from an integer array into the screen buffer.

ENTRY VALUES:

AX = EE3A

DS = Segment address of integer array

DX = Offset address of integer array

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE3AH       ;Transfer screen function

MOV     DX,SEG Array    ;Get segment address of array

MOV     DS,DX           ; ..

MOV     DX,OFFSET Array ;Get offset address of array

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'PutScreen' function in chapter 2 for more information.

********************************************************************************

TITLE:

SetDisplayPage

FUNCTION NUMBER:

59

DESCRIPTION:

Sets the current display page which will display on the computer monitor.

ENTRY VALUES:

AX = EE3B

BX = Display page number

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE3BH       ;Set display page function

MOV     BX,1            ;Set display page number

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'PageCopy' function in chapter 2 for more information on video pages.

********************************************************************************

TITLE:

GetDisplayPage

FUNCTION NUMBER:

60

DESCRIPTION:

Returns the current display page.

ENTRY VALUES:

AX = EE3C

EXIT VALUES:

BX = Current display page number

EXAMPLE:

MOV     AX,0EE3CH       ;Get display page function

INT     10H             ;Call GRAFIX

MOV     [DPage],BX      ;Store current display page number

COMMENTS:

See the 'PageCopy' function in chapter 2 for more information on video pages.

********************************************************************************

TITLE:

SetActivePage

FUNCTION NUMBER:

61

DESCRIPTION:

Sets the current active page which all graphic functions will work with.

ENTRY VALUES:

AX = EE3D

BX = Active page number

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE3DH       ;Set active page function

MOV     BX,1            ;Set active page number

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'PageCopy' function in chapter 2 for more information on video pages.

********************************************************************************

TITLE:

GetActivePage

FUNCTION NUMBER:

62

DESCRIPTION:

Returns the current active page.

ENTRY VALUES:

AX = EE3E

EXIT VALUES:

BX = Current active page number

EXAMPLE:

MOV     AX,0EE3EH       ;Get active page function

INT     10H             ;Call GRAFIX

MOV     [APage],BX      ;Store current active page number

COMMENTS:

See the 'PageCopy' function in chapter 2 for more information on video pages.

********************************************************************************

TITLE:

PageCopy

FUNCTION NUMBER:

63

DESCRIPTION:

Copies the source video page to the destination video page.

ENTRY VALUES:

AX = EE3F

SI = Source video page

DI = Destination video page

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE3FH       ;Page copy function

MOV     SI,3            ;Set source page number

MOV     DI,2            ;Set destination page number

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'PageCopy' function in chapter 2 for more information on video pages.

********************************************************************************

TITLE:

System

FUNCTION NUMBER:

128

DESCRIPTION:

This is  a blanket routine to  incorporate many system level  functions of the

GRAFIX package.

ENTRY VALUES:

AX = EE80

BX = Action

0 = Return residency status in AX

1 = Return current graphics system status in AX

2 = Enter 320x200x16 graphics mode and enable all graphics functions

3 = Enter 80x25 text mode and disable all graphics functions

4 = Disable all current sound processing

5 = Enable sound buffer processing

6 = Disable sound buffer processing

7 = Disable all current noise processing

8 = Enable noise buffer processing

9 = Disable noise buffer processing

10 = Disable all current sound and noise processing

11 = Enter 640x200x16 graphics mode and enable all graphics functions

12 = Return if 640x200x16 color mode is supported

13 = Enter 160x200x16 graphics mode and enable all graphics functions

EXIT VALUES:

BX = 00 entry value

AX = DEAD indicates this package is resident

BX = 01 entry value

AX = 0 indicates graphics system is inactive

1 indicates graphics system is active

BX = 12 entry value

AX = 0 indicates 640x200x16 color graphics mode not supported

1 indicates 640x200x16 color graphics mode supported

Here is a list of each subfunction's description:

BX = 00:

This will enable you to determine if the GRAFIX program is installed or

not. If it is resident the AX register will contain the value 0DEADH to

indicate this  fact. This is  useful to make  sure the program  you are

going to run was loaded after the GRAFIX program.

BX = 01:

This will let you determine the  current status of the graphics system.

The system  must be enabled before  any of it's functions  can be used.

This is a handy little item to have to make sure the graphics system is

enabled before attempting to use any of the functions.

BX = 02:

This enables all functions in the  graphics system and enters the Tandy

1000/PCjr's  320x200x16 color  graphics mode.  It also  resets all  the

graphics variables to their default state.

BX = 03:

This disables all functions in the graphics system and enters the 80x25

text screen mode.

BX = 04:

This  turns all  current sound  processing off.  All current sounds are

turned  off,  the  sound  buffer  is  cleared,  and  sound buffering is

disabled.

BX = 05:

This turns sound buffering on allowing you to make sounds in background

while your program is executing.

BX = 06:

This turns  sound buffering off. The  current sound is left  alone, but

the sound buffer is cleared of any remain sounds to make.

BX = 07:

This  turns all  current noise  processing off.  All current noises are

turned  off,  the  noise  buffer  is  cleared,  and  noise buffering is

disabled.

BX = 08:

This turns noise buffering on allowing you to make noises in background

while your program is executing.

BX = 09:

This turns  noise buffering off. The  current noise is left  alone, but

the noise buffer is cleared of any remain noises to make.

BX = 10:

This  turns all  current noise  and sound  processing off.  All current

noises  and sounds  are turned  off, the  noise and  sound buffers  are

cleared, and noise and sound buffering is disabled.

BX = 11:

This enables all functions in the  graphics system and enters the Tandy

1000  SL/TL's 640x200x16  color graphics  mode. It  also resets all the

graphics variables to their default state.

BX = 12:

This indicates whether the 640x200x16 color graphics mode functions are

supported.

BX = 13:

This enables all functions in the  graphics system and enters the Tandy

1000/PCjr's  160x200x16 color  graphics mode.  It also  resets all  the

graphics variables to their default state.

COMMENTS:

See chapter 6 for the default values for the GRAFIX program.

********************************************************************************

TITLE:

Sound

FUNCTION NUMBER:

129

DESCRIPTION:

Generates a  sound with the  specified frequency, duration,  volume, and sound

channel.

ENTRY VALUES:

AX = EE81

BX = Volume (0-15)

0 is the quietest

15 is the loudest

CX = Sound channel (0-2)

Indicates which one of the three sound channels to use

SI = Frequency (110 - 32767)

DI = Duration (0-65535)

Indicates the number of clock ticks the sound will last

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE81H       ;Sound function

MOV     BX,10           ;Set the volume

MOV     CX,0            ;Set the sound channel

MOV     SI,800          ;Set the frequency

MOV     DI,5            ;Set the duration

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'ExtSound' function in chapter 2 for more information.

********************************************************************************

TITLE:

Noise

FUNCTION NUMBER:

130

DESCRIPTION:

Generates a  periodic or white noise  sound using the specified  type, volume,

and duration.

ENTRY VALUES:

AX = EE82

BX = Type (0-7)

0-3 are periodic noises

4-7 are white noises

CX = Volume (0-15)

0 is the quietest

15 is the loudest

DX = Duration (0-65535)

This is the number of clock ticks the noise will last

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE82H       ;Noise function

MOV     BX,1            ;Set the type

MOV     CX,7            ;Set the volume

MOV     DX,3            ;Set the duration

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'Noise' function in chapter 2 for more information.

********************************************************************************

TITLE:

Delay

FUNCTION NUMBER:

131

DESCRIPTION:

Delays a program the specified number of clock ticks.

ENTRY VALUES:

AX = EE83

BX = Duration (0-65535)

This is the number of clock ticks the delay will last

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE83H       ;Delay function

MOV     BX,8            ;Set the duration

INT     10H             ;Call GRAFIX

COMMENTS:

See the 'Pause' function in chapter 2 for more information.

********************************************************************************

TITLE:

FastTimer

FUNCTION NUMBER:

132

DESCRIPTION:

Sets the speed of the 8253 timer chip.

ENTRY VALUES:

AX = EE84

BX = Speed (2-256)

This is the number of times greater the chip should operate.

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE84H       ;Set 8253 timer chip speed function

MOV     BX,2            ;Set the speed to twice it's normal rate

INT     10H             ;Call GRAFIX

COMMENTS:

See the  'FastTimer' function in chapter  2 for more information  on the 8253

timer chip.

********************************************************************************

TITLE:

ResetTimer

FUNCTION NUMBER:

133

DESCRIPTION:

Resets the 8253 timer chip to it's normal speed which is a setting of 1.

ENTRY VALUES:

AX = EE85

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE85H       ;Reset 8253 timer chip speed function

INT     10H             ;Call GRAFIX

COMMENTS:

See the  'FastTimer' function in chapter  2 for more information  on the 8253

timer chip.

********************************************************************************

TITLE:

GetTimer

FUNCTION NUMBER:

134

DESCRIPTION:

Returns the current speed of the 8253 timer chip.

ENTRY VALUES:

AX = EE86

EXIT VALUES:

BX = Current speed of the 8253 timer chip.

EXAMPLE:

MOV     AX,0EE86H       ;Get 8253 timer chip speed function

INT     10H             ;Call GRAFIX

MOV     [CurSpeed],BX   ;Store current 8253 timer chip speed

COMMENTS:

See the  'FastTimer' function in chapter  2 for more information  on the 8253

timer chip.

********************************************************************************

CHAPTER 4 - GRAFIX Utility Programs

This chapter discusses TANDY11 and ADJMEM the two utility programs provided with

the GRAFIX package.

TANDY11:

Tandy corporation  made excellent computers  in the Tandy  1000 SL/TL/RL. They

had 2  things previous members  of the Tandy  1000 series family  didn't have,

digital  sound and  640x200x16 color  graphics. The  special 640x200x16  color

graphics mode was of special interest to  me since I have been programming the

320x200x16 color  graphics mode for  some time.  When  I got my  Tandy 1000 SL

Technical Reference Manual, I was shocked  that there wasn't even a mention of

this  special  video  mode.  There  was  documentation  for  the digital sound

interface, but not the 640x200x16 color graphics mode. I came across a program

that used this video mode and with DEBUG  I was able to see how to the program

accessed  it. This  limited information  enabled me  to figure  how the  video

memory was set up  and how to program the video system  registers to enter the

640x200x16 color  graphics mode. I was  now able to start  programming in this

special graphics mode.

The reason  I wrote this  program was that  all the other  video modes on  the

Tandy  1000  series  computers  are   supported  by  the  system  BIOS  (Basic

Input/Output  System), but  not the   640x200x16 color  graphics mode.  If the

system BIOS supports the video mode you want  to work in, then you have a full

set of  built in video  functions at your  disposal. These include  entering a

particular video mode, writing characters  to the screen, scrolling the screen

up  or down,  setting the  cursor  position,  etc. Without  them, you  have to

include that logic in every program  you write separately. Seeing this was not

the best situation to  say the least, I wrote a set  of BIOS routines that are

memory resident.  These can be called  just like any other  BIOS routine. They

effectively duplicate the functions included in Tandy's regular video BIOS. So

now  instead of  including scores  of  lines  of Assembler  code to  enter the

640x200x16 color graphics in every program you  write, you can simply do it in

2 lines of Assembly language like this:

MOV     AX,11           ;Set up 640x200x16 color graphics mode

INT     10H             ; ..

I'd say that was a lot easier, wouldn't  you. You can also do it from any high

level language like BASIC, C, or PASCAL as long as that language has access to

BIOS services on your computer. So now to have full access to this video mode,

you only  have to install this  program when you start  up your computer. Just

put the following command in your AUTOEXEC.BAT file:

TANDY11

The  program  verifies  that  the  computer  is  a  Tandy 1000 SL/TL/RL before

attempting to install itself into memory.

ADJMEM:

IBM compatible  computers other than the  Tandy 1000 use a  video adapter that

has the  video memory contained  on the adapter  itself. The Tandy  1000 video

adapter  uses  the  very  top  portion  of  DOS  memory  as video memory. (The

exceptions to this are the Tandy 1000 TX, TL, RL which have the

ability to add an extra 128K of memory which is then used as the video memory.

The  full 640K  of DOS  memory is  then available  at all  times). Please note

ADJMEM does  not apply to  the PCjr as  it maps video  memory from the bottom,

while the Tandy 1000 does it from the top. A PCjr requires a memory management

driver like jrconfig.sys to protect and relocate video memory.

The following table shows the memory requirements for each screen mode used on

the Tandy 1000:

Memory    Video

Mode #    Screen Mode     Colors    Required   Pages

------    -----------     ------    --------   -----

0-1       40x25 Text       16        16K        8

2-3       80x25 Text       16        16K        4

4-5    320x200 Graphics     4        16K        8

6    640x200 Graphics     2        16K        8

8    160x200 Graphics    16        16K        8

9    320x200 Graphics    16        32K        4

10    640x200 Graphics     4        32K        4

* 11    640x200 Graphics    16        64K        2

*  - Tandy  1000 SL/TL/RL  only  supported  through the  use of  the TANDY11

program

The video  pages for the  graphics mode are  based on reserving  128K of video

memory.  The Tandy  1000 has  the ability  to address  this much video memory.

Multiple  video pages  are useful  for high  performance animation. The GRAFIX

programming package supports multiple video pages.

The top 16K of  DOS memory is always protected by BIOS  at start-up. If one of

the  32K or  greater graphics  modes are   used that  area of  memory is  left

unprotected. This can result in a  hung computer if precautions are not taken.

Historically, when you protected that portion of memory the only way to get it

back  was  reboot  your  computer.  That  is  why  I  wrote  this  program. It

dynamically  protects that  extra memory  and then  allows you  to get it back

again.

The  following  batch  file  shows  how  you  could  protect the 32K of memory

necessary  while working  in BIOS  screen modes  9 or  10 and  then release it

again:

ECHO OFF

REM  Protect 32K of video memory

adjmem -16

REM  Run required application

program

REM  Release protected system memory

adjmem +16

Now, instead of  losing that memory while you are  developing your program you

can protect the memory and then release it again without having to reboot your

computer.

To use the program just type ADJMEM +### or ADJMEM -### to add or subtract the

desired K bytes from your Tandy 1000's memory. Here are some examples:

ADJMEM -16      ;Protect 32K of the Tandy 1000's memory

ADJMEM +16      ;Release 16K of the Tandy 1000's memory

ADJMEM -48      ;Protect 64K of the Tandy 1000's memory

ADJMEM +48      ;Release 48K of the Tandy 1000's memory

ADJMEM -112     ;Protect 128K of the Tandy 1000's memory

ADJMEM +112     ;Release 112K of the Tandy 1000's memory

The program accepts any value from -112K  to +112K validating that you are not

adding  or subtracting  more memory  than is  already necessary.  I wrote  the

program to accept this range of values to accommodate the Tandy 1000's ability

to address up to 128K of video  memory. The program verifies that the computer

is a  Tandy 1000 before attempting  to adjust the system  memory. If the Tandy

1000  has  a  128K  memory  expansion  option  installed,  no  memory  will be

protected.

I hope you find this program as useful  as I have. It can really come in handy

if you do a lot Tandy 1000 graphics  programming in one of the 32K video modes

or even the 64K mode on the Tandy 1000 SL/TL.

CHAPTER  5 - Miscellaneous Details About The GRAFIX Package

GRAFIX revision history:

1.0 -  Initial release of GRAFIX.

1.1 -  Added additional functions that allowed specifying the drawing color when

the  function was  called instead  of setting  the current  drawing color

first and then calling the function.

Optimized the following functions:

$Point, $Line, DrawBox, Circle, Paint,  FillBox, and Put.

$Line replaces the $PlotLine function from the previous release.

2.0 -  Added support for the Tandy 1000's 160x200x16 color graphics mode.

Added support  for Tandy 1000  SL/TL/RL 640x200x16 color  graphics mode with

the aid of the TANDY11 driver now supplied with the package.

The package now includes the ADJMEM program for dynamically adjusting the

Tandy 1000's  memory up or  down from the  command line or  a batch file.

This  can be  used to  protect the  video memory  only when necessary and

freeing it up again.

GRAFIX now verifies the computer is a Tandy 1000 series before installing

itself into memory.

Fixed a nasty bug in the Put procedure. Images transferred using the PSET

option on an even pixel address were not transferred properly.

Replaced Circle function. The old  function would cause unexpected divide

by zero errors and crash the computer.

Added  Ellipse function  for more  flexibility than  the Circle  function

could provide in drawing circular shapes.

Added a host of XOR functions for  point  plotting, line  drawing, circle

drawing, box drawing, and box filling.

Added definable viewport. All  graphics functions except PrintString will

clip on the viewport boundaries.

Added  'GetScreen'  and  'PutScreen'  functions  for  ultra fast save and

restore of the video buffer.

Speeded up the majority of graphic functions by 2-3 times than in version

1.1.

2.5 -  Added   video   paging   functions:   'SetActivePage',   'GetActivePage',

'SetDisplayPage', 'GetDisplayPage', and 'PageCopy'.

Rewrote sound and noise systems for better quality sound and performance.

Added  8253 timer  functions: 'FastTimer',  'ResetTimer', and 'GetTimer'.

The 'FastTimer'  function increases the  precesion of the  amount of time

sounds, noises, and delays will last.

Included new command line parameter /M  which can be used to indicate how

much  video  memory  should  be  protected  on  start-up. GRAFIX will now

protect  up  to  128K  of  video  memory.  Memory  protection  is skipped

automatically if the Tandy 1000 has a 128K memory expansion option.

Changed  'ADJMEM' program  to protect   up 128K  of video  memory. Memory

protection is skipped  automatically if the Tandy 1000  has a 128K memory

expansion option.

ADJMEM now verifies  the computer is a Tandy 1000  before it will attempt

to protect any memory.

2.55 - Decrease memory useage of GRAFIX.COM by over 5,000 bytes.

Made minor updates to a few demo programs.

2.60 - Fixes all know bugs in prior releases.  TANDY11 is now at version 1.1.  It

corrects a problem after 640x200x16 color mode is used which causes screen

corruption in the other graphics modes.

2.70 - Support has now been added for  the IBM PCjr for both graphics and sound.

Made small changes to Tandy11 to include wording the Tandy 1000 RL in the

source code and TANDY11.COM driver. Including JRAUD3V.COM and JRAUDRST.COM

for routing PCjr audio.

--------------------------------------------------------------------------------

The following are the default values for GRAFIX program:

Current X, Y position:  X = 0

Y = 0

Current text position: Row = 1

Column = 1

Viewport size for      Minimum X = 0

160x200x16 mode:       Minimum Y = 0

Maximum X = 159

Maximum Y = 199

Viewport size for      Minimum X = 0

320x200x16 mode:       Minimum Y = 0

Maximum X = 319

Maximum Y = 199

Viewport size for      Minimum X = 0

640x200x16 mode:       Minimum Y = 0

Maximum X = 639

Maximum Y = 199

Aspect ratio for       Horizontal = 3

160x200x16 mode:       Vertical = 5

Aspect ratio for       Horizontal = 6

320x200x16 mode:       Vertical = 5

Aspect ratio for       Horizontal = 12

640x200x16 mode:       Vertical = 5

Active page:           0

Display page:          0

Drawing color:         1 (Blue)

Text color:            15 (Light White)

Background color:      0 (Black)

Graphics cursor:       Off

Graphics cursor size:  Full

Sound buffering:       Off

Noise buffering:       Off

--------------------------------------------------------------------------------

The following list are the allowable  values for all graphics functions for each

graphics mode:

160x200x16 Color Graphics Mode

------------------------------

X           = 0 - 159

Y           = 0 - 199

Text Column = 1 -  20

Text Row    = 1 -  25

Color       = 0 -  15

Page number = 0 -   7

320x200x16 Color Graphics Mode

------------------------------

X           = 0 - 319

Y           = 0 - 199

Text Column = 1 -  40

Text Row    = 1 -  25

Color       = 0 -  15

Page number = 0 -   3

640x200x16 Color Graphics Mode

------------------------------

X           = 0 - 639

Y           = 0 - 199

Text Column = 1 -  80

Text Row    = 1 -  25

Color       = 0 -  15

Page number = 0 -   1

--------------------------------------------------------------------------------

Some final items:

GRAFIX by default will protect the top 32K of DOS memory when it is installed.

The default is 64K on a Tandy 1000 SL/TL/RL. The 'ADJMEM' section in chapter 4

explains why this is necessary. Memory is not protected if the Tandy 1000 is a

TX, TL,  or RL with a  128K memory expansion option.  Memory protection can be

skipped by installing GRAFIX with the /N option like this:

GRAFIX /N

You can specify the amount of video memory you want to protect by using the /M

option. This is useful if you want to protect more than 32K or 64K to do video

paging safely. The  /M option will accept any  value from 16K to 128K.  If you

wanted GRAFIX to protect 128K of video memory, just install GRAFIX like this:

GRAFIX /M128

A few notes on  the PCjr. You can use two included  programs for routing audio

on the PCjr.  JRAUD3V.COM routes the 3 voice sound  to through the monitor and

RCA audio  jack. JRAUDRST.COM resets the  audio back it's default  values. You

never need to do this on Tandy 1000 as all audio is routed differently. GRAFIX

may work on a 128K PCjr, but I  recommend a 256K PCjr or better using a memory

management driver like jrconfig.sys.

I have spent well over 2 years  writing the GRAFIX package and have enjoyed it

very much. I previously asked for a $15.00 register fee for using the package.

I found the  shareware market to less than profitable.  There are many reasons

for this.  Tandy 1000 16  color graphics are  pretty much a  thing of the past

with Super VGA  adapters as cheap as they  are. Only a small number  of people

who own a Tandy 1000 are going to  do any kind of serious programming on it. I

myself have bought a 386 20Mhz  computer with Super VGA. All things considered

then, I am making GRAFIX public domain including all the source code. The only

thing I  ask is that  my name and  copyright must be  included no matter  what

changes are made.